RF THROUGHPUT ANALYSIS

UNIVERSAL REFERENCE DOCUMENT

Comprehensive RAG Knowledge Base for DAS Speed Test Validation

Covers: LTE (Bands 2/4/5/12/25/41/66/71) | NR SA & NSA (n25/n41/n71/n77/n78/n258/n260/n261)

DAS Architectures: Analog | Digital | Hybrid | C-RAN | O-RAN | Small Cell

Includes: Thresholds | Formulas | Diagnostic Trees | Troubleshooting | Improvement Steps

Version 1.3  |  Expert RF Engineering Reference  |  T-Mobile DAS Program

TABLE OF CONTENTS

1. Purpose & Scope	6

2. RF Parameter Encyclopedia	7

2.1 RSRP (Reference Signal Received Power)	7

2.1.1 Definition & Measurement	7

2.1.2 Thresholds	7

2.1.3 RSRP-to-Throughput Relationship	7

2.1.4 Troubleshooting: RSRP Outside Threshold	7

2.1.5 RSRP Improvement Actions	8

2.2 SINR / SNR (Signal-to-Interference-plus-Noise Ratio)	9

2.2.1 Definition & Measurement	9

2.2.2 Thresholds	9

2.2.3 Troubleshooting: SINR Below Threshold	9

2.2.4 SINR Improvement Actions	9

2.3 RSRQ (Reference Signal Received Quality)	11

2.3.1 Definition & Measurement	11

2.3.2 Thresholds & Impact	11

2.3.3 Troubleshooting: RSRQ Outside Threshold	11

2.4 UE TX Power	12

2.4.1 Definition & Measurement	12

2.4.2 Thresholds & UL Throughput Impact	12

2.4.3 Troubleshooting: TX Power Issues	12

2.4.4 TX Power Improvement Actions	12

2.5 BLER (Block Error Rate)	14

2.5.1 Definition & Measurement	14

2.5.2 Thresholds & Throughput Impact	14

2.5.3 Troubleshooting: BLER Above 10%	14

2.6 NR DL Scheduling	16

2.6.1 Definition	16

2.6.2 Expected Values	16

2.6.3 Throughput Formula	16

2.6.4 Troubleshooting Low Scheduling	16

2.7 Channel Bandwidth & PRB Count	17

2.7.1 Definition	17

2.7.2 Complete PRB Reference Table	17

2.7.3 Bandwidth Verification From CIQ	17

2.8 MIMO, Antenna Configuration & Rank Indicator	18

2.8.1 DAS MIMO Configurations	18

2.8.2 NR Receive Chain Diagnostics (RX0-RX3)	18

2.8.3 NR_ANT MAX RSRP vs NR_ANT MIN RSRP	18

2.8.4 Troubleshooting MIMO Issues	18

2.9 EN-DC, Carrier Aggregation & Dual Connectivity	19

2.9.1 EN-DC Architecture	19

2.9.2 LTE Anchor Impact on NR Throughput	19

2.9.3 CA-Act (Carrier Aggregation Active)	19

2.9.4 NR CA Status	19

2.9.5 Troubleshooting EN-DC / CA	19

2.10 Additional Service Mode Parameters	21

2.10.1 NR_SCS (Subcarrier Spacing)	21

2.10.2 NR_CDRX (Connected Mode DRX)	21

2.10.3 NR_Tx Phy Channel Type	21

2.10.4 NR_SLICE/PDU	21

2.10.5 Paging Cycle	21

2.10.6 EARFCN / NR_ARFCN	21

2.10.7 PCI (Physical Cell Identity)	21

2.11 TDD Configuration (n41/n77/n78)	23

2.11.1 Why TDD Matters for Throughput	23

2.11.2 Common TDD Slot Patterns	23

2.11.3 Special Slot	23

2.11.4 TDD Troubleshooting	23

2.12 RLF, RRC State & Link Stability	24

2.12.1 NR_RLF Count / NR_SA_RLF Count	24

2.12.2 NR_RRC State	24

2.12.3 ASDiv (Antenna Switching Diversity)	24

2.12.4 NR-DC SCG Not Present	24

2.13 UL-Specific Throughput Factors	25

2.13.1 UL Throughput Formula	25

2.13.2 UL Throughput Ceiling Factors (Priority Order)	25

2.13.3 UL CA (Uplink Carrier Aggregation)	25

2.15 Hardware Components: Radio Units, Baseband & Antenna	26

2.15.1 Baseband Units	26

2.15.2 Radio Units	26

2.15.3 Radio Unit Impact on Throughput	26

2.15.4 Antenna System	27

2.15.5 Hardware Configuration String (CIQ)	27

2.15.6 Hardware Troubleshooting	28

2.16 Physical Layer / Antenna System Thresholds

2.16.1 RSSI Hot/Cold Thresholds

2.16.2 RSSI Target (AILG ON)

2.16.3 RSSI Imbalance (Branch Delta)

2.16.4 VSWR / Return Loss

2.16.5 Fiber Loss & Rx Power

3. Speed Test Throughput Thresholds by BW & Layer	29

3.1 SISO Configuration	29

3.2 MIMO Configuration	29

4. Diagnostic Decision Trees	30

4.1 DL Throughput Below Expected	30

4.2 UL Throughput Below Expected	30

5. Speed Test Execution Best Practices	32

5.1 Service Mode - Speed Test Timestamp Rule	32

5.2 Screenshot Labeling Convention	32

5.3 TX Power Verification	32

5.4 NR Testing Sequence	32

5.5 Common Pitfalls	32

6. Essential Formulas & Calculations	33

6.1 Theoretical Peak Throughput	33

6.2 Throughput Efficiency	33

6.3 UL Throughput Estimation	33

6.4 ENDC Combined Throughput	33

6.5 Path Loss Budget	33

6.6 SINR Estimation from RSRP	33

7. DAS Vendor-Specific Throughput Analysis	39

7.1 DAS Throughput Baselines (All Vendors)	39

7.2 CommScope (ERA, ION-E, FlexWave)	39

7.3 JMA Wireless (TEKO, NCHA)	40

7.4 ADRF (SDR, ADX)	41

7.5 Corning (Everon, ONE, SpiderCloud)	41

7.6 SOLiD (ALLIANCE, GENESIS)	42

7.7 Universal DAS Throughput Symptom Matrix	42

7.8 DAS Escalation Path	42

8. Comprehensive Glossary	43

9. MS2 KPI & Counter Correlation Framework	51

9.1 NR KPI Correlation to RF Parameters	51

9.1.1 Throughput KPIs	51

9.1.2 Accessibility KPIs	51

9.1.3 Retainability KPIs	52

9.1.4 Interference & Power KPIs	52

9.1.5 Mobility KPIs	53

9.1.6 Capacity & User KPIs	53

9.2 LTE KPI Correlation to RF Parameters	53

9.2.1 LTE Throughput & Traffic KPIs	53

9.2.2 VoLTE KPIs (LTE Anchor Voice Quality)	54

9.2.3 E-RAB (Bearer) KPIs	54

9.2.4 RACH & Access KPIs	54

9.2.5 MIMO & Carrier Aggregation KPIs	55

9.2.6 LTE Mobility KPIs	55

9.3 CIQ Parameter → KPI Impact Chain	55

9.3.1 Power & Coverage Parameters	55

9.3.2 EN-DC & CA Parameters	56

9.3.3 TDD & Scheduling Parameters	56

9.4 Expert RF Engineer’s KPI Monitoring Playbook for DAS Validation	56

9.4.1 Pre-Test KPI Baseline (T-24h)	56

9.4.2 During-Test KPI Correlation	57

9.4.3 Post-Test KPI Validation (T+24h)	57

# 1. Purpose & Scope

This document serves as the universal RAG (Retrieval-Augmented Generation) knowledge base for RF throughput analysis on any T-Mobile DAS site. It provides the complete technical framework for: (a) interpreting every RF parameter visible in Samsung Galaxy service mode screenshots, (b) correlating those parameters to DL and UL throughput, (c) identifying pass/fail/warning conditions against MOP thresholds, (d) diagnosing root causes of throughput degradation, and (e) prescribing corrective actions.

This document is technology-agnostic within the cellular domain and covers LTE, LTE-Advanced, NR SA, NR NSA (EN-DC), NR CA, and future NR-DC architectures. It is intended for use by field engineers, RF optimization engineers, back-office IX teams, and AI/ML systems performing automated validation.

# 2. RF Parameter Encyclopedia

Every parameter observable in Samsung service mode is documented below with its definition, measurement methodology, units, valid ranges, impact on throughput, warning thresholds, failure thresholds, troubleshooting steps, and improvement actions.

## 2.1 RSRP (Reference Signal Received Power)

### 2.1.1 Definition & Measurement

RSRP is the linear average power (in dBm) of the resource elements carrying cell-specific reference signals (CRS for LTE, SS-RSRP for NR SSB, CSI-RSRP for NR CSI-RS) within the considered measurement bandwidth. It is measured per antenna port and the UE reports the highest value across receive antennas.

RSRP (dBm) = Total received power on RS REs averaged across measurement bandwidth

Path Loss (dB) = eNB/gNB TX Power (dBm) + Antenna Gain (dBi) - Cable Loss (dB) - RSRP (dBm)

Samsung service mode displays: RSRP:-XX for LTE | NR5G_RSRP: -XX for NR | NR_ANT MAX RSRP / NR_ANT MIN RSRP for per-antenna NR values | NR RX0-RX3 RSRP for individual receive chain values.

### 2.1.2 Thresholds

| Test Location | Technology | Range (dBm) | PASS | FAIL | Impact on Throughput |
| --- | --- | --- | --- | --- | --- |
| Radio End | LTE | -60 to -40 | In range | Outside | 256QAM sustainable |
| Radio End | NR | -60 to -40 | In range | Outside | Max MCS achievable |
| Antenna End | LTE | -75 to -50 | In range | Outside | 64-256QAM range |
| Antenna End | NR | -75 to -50 | In range | Outside | 64-256QAM range |

Note: These thresholds apply to all FR1 NR bands (n25, n41, n71, n77/n78). For FR2 (mmWave) RSRP thresholds, see Section 2.18.2. n77/n78 C-Band may show 3-5 dB lower RSRP than n41 at equivalent DAS configuration due to higher free-space path loss.

### 2.1.3 RSRP-to-Throughput Relationship

RSRP determines the Channel Quality Indicator (CQI) which maps to Modulation and Coding Scheme (MCS). The relationship is not linear but follows a step function as modulation orders change:

| RSRP Range | Condition | Typical CQI | Modulation | Bits/Symbol | DL Efficiency |
| --- | --- | --- | --- | --- | --- |
| > -65 dBm | Excellent | 13-15 | 256QAM | 8 | 100% |
| -75 to -65 | Good | 10-12 | 64QAM | 6 | ~75% |
| -85 to -75 | Fair | 7-9 | 16QAM | 4 | ~50% |
| -95 to -85 | Poor | 4-6 | QPSK | 2 | ~25% |
| < -95 dBm | Very Poor | 1-3 | QPSK low rate | 2 | <15% |

KEY INSIGHT: Every 3 dB RSRP improvement roughly doubles SNR in noise-limited environments. At each modulation boundary crossing (QPSK to 16QAM, 16QAM to 64QAM, 64QAM to 256QAM), throughput jumps ~33%. Between boundaries, throughput scales with coding rate changes (~5-15% per CQI step).

### 2.1.4 Troubleshooting: RSRP Outside Threshold

RSRP too weak (below -75 dBm at antenna end):

- Step 1: Verify the UE is locked to the correct serving cell PCI. Cross-reference with CIQ PCI allocation. Wrong-cell camping causes apparent low RSRP.

- Step 2: Check DAS head-end output power. Verify attenuator settings and amplifier gain at the BDA/Remote Unit.

- Step 3: Measure cable/fiber loss from head-end to the antenna under test. Compare with CIQ-specified loss budget. Excessive loss indicates damaged cable or bad connector.

- Step 4: Inspect antenna installation. Verify antenna type matches design (omnidirectional vs. directional), mounting orientation, and that no physical obstructions have been introduced.

- Step 5: For NR, check NR_ANT MAX RSRP vs NR_ANT MIN RSRP. If MIN is -140 dBm, one antenna path is dead (cable, splitter, or connector issue).

FIX: Increase DAS output power by adjusting amplifier gain (typical range: +3 to +6 dB). If gain is already at max, reduce attenuator pads. If path loss exceeds budget, replace damaged coaxial cable segments.

RSRP too strong (above -40 dBm at radio end):

- Indicates UE is too close to the dummy antenna or DAS output power is excessively high.

- Step 1: Increase distance from dummy antenna by 1-2 feet.

- Step 2: If at antenna end, the DAS may be over-driving. Add attenuation or reduce amplifier gain to bring RSRP into -50 to -60 range.

FIX: Over-powered DAS antennas cause ADC saturation at the UE receiver, which paradoxically degrades SINR and throughput. Optimal RSRP at antenna end is -55 to -65 dBm.

### 2.1.5 RSRP Improvement Actions

- Replace damaged coaxial cables or connectors showing >1 dB excess loss beyond specification.

- Re-torque all N-type and 7-16 DIN connectors to manufacturer specification (typically 15-20 ft-lbs).

- Verify DAS system gain by injecting a known signal at the donor port and measuring at each antenna port.

- For multi-sector DAS, ensure equal power distribution. Unbalanced splitter outputs cause sector-to-sector RSRP variance.

- Consider adding antennas to improve coverage in dead zones rather than increasing power (which raises interference).

KPI CORRELATION: When RSRP is outside threshold, MS2 KPIs degrade: RACH CBRA Success Rate (NTM_NR_ACC_03_02_023 / RACHCBRASuccessRate) drops; NSA SN Addition Failure Rate (NTM_NR_ACC_03_02_096) increases; DL Average UE Throughput (NTM_NR_CAP_03_04_061) decreases due to lower CQI/MCS. Monitor PMRADIORACBFAILMSG1OOC counter for Msg1 out-of-coverage failures as a direct indicator of UL coverage gaps.

## 2.2 SINR / SNR (Signal-to-Interference-plus-Noise Ratio)

### 2.2.1 Definition & Measurement

SINR is the ratio of the desired signal power to the total interference-plus-noise power, expressed in dB. It is the single most critical parameter for instantaneous throughput prediction because it directly determines the maximum sustainable MCS index.

SINR (dB) = 10 * log10(P_signal / (P_interference + P_noise))

Shannon Capacity: C = BW * log2(1 + SINR_linear) [bps], where BW is in Hz

Samsung service mode displays: SNR:XX.X for LTE | NR5G_SINR: XX.X for NR. Note: Samsung labels this 'SNR' for LTE and 'SINR' for NR, but both represent the same physical measurement.

### 2.2.2 Thresholds

| SINR (dB) | Rating | LTE MCS | NR MCS | Max Modulation | DL Throughput % |
| --- | --- | --- | --- | --- | --- |
| > 30 | Excellent | 28 | 27 (256QAM) | 256QAM R0.93 | 100% |
| 25-30 | Very Good | 25-27 | 24-26 | 256QAM R0.75-0.88 | 85-95% |
| 20-25 | Good | 22-24 | 20-23 | 256QAM low CR / 64QAM high CR | 70-85% |
| 15-20 | Moderate | 17-21 | 15-19 | 64QAM mid rate | 55-70% |
| 10-15 | Fair | 12-16 | 10-14 | 16QAM | 35-55% |
| 3-10 | Poor | 5-11 | 4-9 | QPSK high rate | 15-35% |
| < 3 | Very Poor | 0-4 | 0-3 | QPSK low rate | <15% |

MOP Pass Threshold: Above 25 dB for both Radio End and Antenna End tests.

### 2.2.3 Troubleshooting: SINR Below Threshold

SINR below 25 dB in DAS environment (should typically be 25-40 dB):

- Step 1: Check for macro interference. Walk to a window and note if RSRP from external macro cells is stronger than DAS RSRP. If macro RSRP > DAS RSRP - 6 dB, the macro is creating co-channel interference.

- Step 2: Verify PCI planning. PCI collision or confusion between DAS cells and neighboring macro cells causes massive SINR degradation. Cross-reference PCI in service mode with CIQ PCI allocation and macro PCI database.

- Step 3: Check for PIM (Passive Intermodulation). PIM generates in-band interference from rusty connectors, loose hardware, or dissimilar metals in the RF path. PIM typically manifests as 3-10 dB SINR degradation with no change in RSRP.

- Step 4: Inspect DAS amplifiers for saturation. Over-driven amplifiers generate intermodulation products that appear as broadband noise. Check amplifier input power vs. rated maximum input.

- Step 5: Verify frequency isolation between DAS bands. Poor filter performance allows inter-band leakage that raises the noise floor on adjacent bands.

- Step 6: For NR, check if the UE is camping on the correct SSB beam (NR_SSB_Index). Incorrect beam selection degrades SINR.

### 2.2.4 SINR Improvement Actions

FIX: PIM remediation: Replace all connectors showing corrosion. Torque all connections to spec. Replace any jumper cables older than 5 years. Use PIM-rated components (<-153 dBc) throughout the RF path.

- Increase DAS dominance: raise DAS output power or add antennas to ensure DAS RSRP exceeds macro by >10 dB at all test points.

- Add isolation: install RF shielding (Low-E glass, metallic window film) to reduce macro ingress by 15-25 dB.

- Optimize PCI planning: ensure no PCI mod-3 collision between co-channel DAS cells and proximate macro cells.

- For multi-sector DAS, verify sector isolation. Insufficient isolation between sectors causes intra-system interference.

- Check DAS noise figure. Cascaded noise figure should be <6 dB for typical DAS. High noise figure directly reduces SINR.

KPI CORRELATION: When SINR is below threshold, MS2 KPIs degrade: PUSCH Receive Interference Power (NTM_NR_PWR_03_15_010) increases; DL/UL Average UE Throughput drops due to MCS reduction; NSA SN Total Drop Rate (NTM_NR_RET_03_03_001) increases as sustained low SINR triggers RLF. PCI mod-3 collisions also impact SA HO Intra-Frequency Success Rate (NTM_NR_MOB_03_05_103) at sector boundaries.

## 2.3 RSRQ (Reference Signal Received Quality)

### 2.3.1 Definition & Measurement

RSRQ (dB) = 10*log10(N) + RSRP(dBm) - RSSI(dBm),  where linear: RSRQ = N * RSRP_linear / RSSI_linear

Where N is the number of resource blocks in the measurement bandwidth. RSRQ captures the ratio of useful signal to total received power including all interference, noise, and serving cell data power. It is particularly valuable for handover decisions and for detecting wideband interference that SINR may not fully capture.

Samsung displays: RSRQ:-XX for LTE | NR5G RSRQ: -XX for NR. Range is typically -3 (best) to -20 (worst).

### 2.3.2 Thresholds & Impact

MOP Pass Range: -12 to -3 (both Radio End and Antenna End). Values of -3 to -5 indicate an extremely clean environment. Values of -10 to -12 indicate moderate interference or high cell loading. Below -12 indicates severe interference or the UE is at cell edge.

Throughput impact: RSRQ below -12 triggers handover attempts, causing throughput interruptions. RSRQ between -10 and -12 causes conservative MCS selection due to perceived poor channel quality. RSRQ above -6 allows maximum MCS and throughput.

### 2.3.3 Troubleshooting: RSRQ Outside Threshold

- RSRQ worse than -12: Primary cause is interference. Follow SINR troubleshooting steps (Section 2.2.3). Additionally, check for UE camping on a distant cell where RSRP is low but interference from nearer cells is high.

- RSRQ better than -3: Not typically a problem, but may indicate the UE is excessively close to the antenna with very high RSRP. Can cause AGC issues.

- RSRQ oscillating rapidly: Indicates the UE is at a cell boundary experiencing ping-pong between cells. Check handover hysteresis and cell reselection parameters.

FIX: For consistently poor RSRQ, the solution is usually to improve RSRP (increasing signal) or reduce interference (PCI optimization, power balancing, isolation improvement).

## 2.4 UE TX Power

### 2.4.1 Definition & Measurement

UE TX Power is the handset transmit power in dBm. It is controlled by the open-loop and closed-loop power control algorithms. The UE adjusts TX power based on path loss estimation (open-loop) and TPC commands from the eNB/gNB (closed-loop) to achieve the target received power at the base station.

LTE UL TX Power = min(P_max, P_0 + alpha*PL + delta_TF + f(i)) [dBm]

where: P_max = UE max power (23 dBm), P_0 = target, alpha = path loss compensation, PL = path loss

Samsung displays: Tx Pwr: -XX dBm for LTE | NR_Tx Pwr: -XX dBm for NR | ENDC Total Tx Pwr: -XX dBm for combined LTE+NR uplink.

### 2.4.2 Thresholds & UL Throughput Impact

MOP Requirement: Always Negative. A positive TX power in a DAS environment indicates the UE is power-limited, which directly caps UL throughput.

| TX Power Range | Condition | Path Loss Est. | UL MCS Impact | UL Throughput | Battery Impact |
| --- | --- | --- | --- | --- | --- |
| < -20 dBm | Excellent | Very low PL | Max MCS | 100% | Minimal drain |
| -20 to -5 | Good | Low-moderate PL | High MCS | 85-100% | Low drain |
| -5 to 0 | Marginal | Moderate PL | Moderate MCS | 70-85% | Moderate drain |
| 0 to +10 | Warning | High PL | Reduced MCS | 40-70% | High drain |
| +10 to +23 | Power-Limited | Very high PL | Minimum MCS | <40% | Max drain |

### 2.4.3 Troubleshooting: TX Power Issues

- TX Power positive (>0 dBm): UE is compensating for excessive path loss. Check DAS UL path: verify UL gain is configured correctly, check for failed UL amplifiers, inspect antenna VSWR.

- TX Power near 0 dBm on NR while LTE TX is very negative: Indicates DAS power imbalance between LTE and NR UL paths. The NR remote unit may have lower UL sensitivity.

- ENDC Total TX much higher than individual NR TX: Normal in NSA mode. The LTE anchor UL is consuming significant power budget. If ENDC > 0, prioritize LTE anchor path optimization.

FIX: For high TX power: Increase DAS UL gain by adjusting remote unit uplink attenuators. Verify uplink noise figure of the DAS system. Ensure bidirectional amplifiers are functioning on all paths.

### 2.4.4 TX Power Improvement Actions

- Optimize P_0 (target received power) at the eNB: lower P_0 allows the UE to transmit at lower power while still meeting the UL SINR target.

- Increase alpha (path loss compensation factor) from fractional (0.8) to full (1.0) for DAS cells where all users are relatively close.

- Verify DAS UL system gain chain: from antenna to BDA to BTS, ensure each stage has correct gain setting.

- For ENDC: if LTE anchor consumes excessive TX power, optimize LTE anchor path loss to free up power headroom for NR UL.

EN-DC UE TX Power Split: The UE total TX power (23 dBm) is dynamically split between LTE and NR uplinks per 3GPP P-MPR (power management maximum power reduction). LTE anchor typically gets priority. If LTE consumes 20 dBm, only ~3 dBm headroom remains for NR UL, severely limiting NR UL throughput and MCS.

KPI CORRELATION: Positive UE TX Power impacts: UL Average UE Throughput (NTM_NR_CAP_03_04_062) constrained; RACH CBRA Success Rate degrades when TX Power headroom exhausted; NR UL MAC Traffic Volume trends downward. On LTE anchor, high TX Power drives VoLTE Drop Rate up. Cross-reference VOLTE_Drops_due_to_UELost — UE Lost drops with positive TX Power = confirmed UL coverage hole.

## 2.5 BLER (Block Error Rate)

### 2.5.1 Definition & Measurement

BLER is the ratio of erroneously received transport blocks to total received transport blocks, expressed as a percentage. The NR/LTE scheduler uses outer-loop link adaptation (OLLA) to adjust MCS such that the first-transmission BLER target is approximately 10%. This is a deliberate design choice: running at 10% BLER maximizes spectral efficiency because HARQ retransmissions recover most errors while allowing aggressive MCS.

BLER = (Number of errored transport blocks / Total transport blocks) * 100%

Effective throughput = Raw throughput * (1 - BLER) * (1 + retransmission_overhead)^(-1)

Samsung displays: NR_BLER: XX.XX for NR. LTE BLER is not directly shown in Samsung service mode but can be inferred from DL throughput relative to theoretical peak.

### 2.5.2 Thresholds & Throughput Impact

| BLER % | Rating | Scheduler Behavior | HARQ Impact | DL Throughput Impact | Latency Impact |
| --- | --- | --- | --- | --- | --- |
| 0-2% | Conservative | MCS too low (underutilized) | Minimal retransmissions | -5 to -15% vs optimal | Low |
| 2-5% | Good | MCS well-tuned | Occasional retransmissions | -2 to -5% | Low |
| 5-10% | Optimal | OLLA converged at target | ~1 in 10-20 blocks retransmitted | Maximum efficiency | Slight increase |
| 10-15% | Elevated | OLLA attempting to reduce MCS | ~1 in 7-10 blocks | -5 to -15% | Moderate |
| 15-20% | High | MCS aggressively reduced | ~1 in 5-7 blocks | -15 to -30% | High |
| > 20% | Critical | Near-minimum MCS | Retransmission storm | > -30% | Very high |

KEY INSIGHT: A BLER of 0% is NOT optimal. It means the scheduler is using too conservative an MCS, leaving capacity on the table. The ideal operating point is 5-10% first-transmission BLER.

Note: After handover or initial RACH, OLLA requires 50-200 ms to converge to the 10% BLER target. During convergence, the scheduler uses conservative MCS, contributing to the "ramp-up" effect visible in speed test throughput graphs. This is normal behavior and not a DAS issue.

### 2.5.3 Troubleshooting: BLER Above 10%

- Step 1: Correlate with SINR. If SINR is also degraded, the BLER is caused by RF conditions (interference or weak signal). Address SINR root cause first.

- Step 2: If SINR is good (>25 dB) but BLER is high, suspect DAS linearity issues. Non-linear DAS components (amplifiers near compression, PIM) cause EVM degradation that directly increases BLER without significantly affecting SINR.

- Step 3: Check for frequency-selective fading. In-building reflections can cause notches in the channel frequency response. NR with 100 MHz bandwidth is particularly susceptible.

- Step 4: Verify DAS EVM (Error Vector Magnitude) performance. DAS systems should deliver EVM < 8% for 256QAM. Measure EVM at the antenna port using a signal analyzer.

- Step 5: Check for timing issues. In TDD systems, DL-to-UL switching point alignment errors cause block errors at the transition boundary.

FIX: If BLER is consistently 10-15% with good SINR, the issue is likely EVM or linearity. Replace suspected amplifiers. If BLER is variable (swinging 0-20%), suspect intermittent PIM or loose connector.

KPI CORRELATION: BLER above 10% with good SINR (> 25 dB) is a DAS linearity issue. Network-side confirmation: DL Average UE Throughput (NTM_NR_CAP_03_04_061) degraded despite acceptable PUSCH SINR. SA DRB Active Drop Rate (NTM_NR_RET_03_03_019) may increase if BLER exceeds 20% sustained. Track ERABDropduetoUELost on LTE — EVM-related BLER on the anchor degrades VoLTE retention.

## 2.6 NR DL Scheduling

### 2.6.1 Definition

NR_DL_Scheduling represents the percentage of downlink time-domain resources (slots) being allocated for PDSCH data transmission. The remaining resources are consumed by: SSB (synchronization signal blocks), CORESET (control resource set), CSI-RS (channel state information reference signals), SRS (sounding reference signals), guard periods (TDD), and HARQ feedback slots.

### 2.6.2 Expected Values

NR SA mode: 85-100% during active speed test (minimal overhead). NR NSA (EN-DC): 65-75% during active speed test (additional overhead from LTE-NR coordination, cross-carrier scheduling, and split-bearer management). Values below 60% indicate scheduling inefficiency or excessive control overhead. A value near 0% during service mode capture (before speed test starts) is normal and expected.

### 2.6.3 Throughput Formula

Achievable DL Throughput (Mbps) = N_PRB * 12 * 14 * Mod_bits * CR * (Sched%/100) * N_layers * (1-OH%) * Slots_per_sec / 1e6

Slots_per_sec: 15kHz SCS = 1000, 30kHz SCS = 2000, 60kHz SCS = 4000, 120kHz SCS = 8000

Example for 100 MHz n41 (30 kHz SCS): 273 PRBs * 12 * 14 * 8 (256QAM) * 0.93 (CR) * 2 (layers) = 682,780 bits/slot. At 2000 slots/sec = 1,365 Mbps raw. Apply 0.70 scheduling (NSA) = 956 Mbps. Apply 0.90 overhead = 860 Mbps theoretical ceiling. Observed 598-776 Mbps represents 70-90% of this ceiling, with the gap attributable to BLER retransmissions, imperfect MCS (not always peak 256QAM), and real-world scheduling gaps.

KEY INSIGHT: CRITICAL: For TDD bands (n41, n77, n78), the DL throughput is further reduced by the TDD DL-UL slot ratio. T-Mobile n41 typically uses DDDSU pattern (3 DL : 1 Special : 1 UL per 5-slot frame) giving ~70% DL slots. This TDD ratio is ALREADY embedded in the ~70% NR_DL_Scheduling observed in service mode.

### 2.6.4 Troubleshooting Low Scheduling

- Scheduling < 50% during active data transfer: Check for eNB/gNB resource contention (other active users consuming scheduler resources). Verify no admission control throttling.

- Scheduling 0% or near-0%: Screenshot taken during idle period. Retake during active speed test. Ensure speed test is running on the NR path (verify 5G icon in status bar).

- SA scheduling significantly higher than NSA: This is expected. NSA overhead can consume 25-35% of resources. If the gap is > 40%, investigate EN-DC coordination timer configuration.

FIX: To improve NR DL scheduling in NSA, optimize the X2/Xn interface latency between eNB and gNB. Reduce ENDC split-bearer buffering. Consider migrating to SA mode if available.

## 2.7 Channel Bandwidth & PRB Count

### 2.7.1 Definition

Channel bandwidth determines the total number of Physical Resource Blocks (PRBs) available for scheduling. Each PRB consists of 12 subcarriers in the frequency domain. Bandwidth is the single most impactful parameter for peak throughput as it linearly scales capacity.

### 2.7.2 Complete PRB Reference Table

| BW (MHz) | LTE PRBs | NR 15kHz | NR 30kHz | NR 60kHz | NR 120kHz | Peak DL SISO | Peak DL 2x2 | Peak DL 4x4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | 25 | 11 | -- | -- | ~37 | ~75 | ~150 |
| 10 | 50 | 52 | 24 | 11 | -- | ~75 | ~150 | ~300 |
| 15 | 75 | 79 | 38 | 18 | -- | ~110 | ~220 | ~440 |
| 20 | 100 | 106 | 51 | 24 | -- | ~150 | ~300 | ~600 |
| 40 | -- | 216 | 106 | 51 | 24 | ~300 | ~600 | ~1200 |
| 60 | -- | -- | 162 | 79 | 38 | ~450 | ~900 | ~1800 |
| 80 | -- | -- | 217 | 107 | 51 | ~600 | ~1200 | ~2400 |
| 90 | -- | -- | 245 | 121 | 58 | ~670 | ~1340 | ~2680 |
| 100 | -- | -- | 273 | 135 | 66 | ~750 | ~1500 | ~3000 |

Peak DL values assume 256QAM, maximum coding rate, 100% scheduling, 0% BLER (theoretical). Real-world DAS typically achieves 50-80% of theoretical peak.

### 2.7.3 Bandwidth Verification From CIQ

Always cross-reference the bandwidth displayed in service mode (BW:XX) with the CIQ dlChannelBandwidth value (expressed in kHz, divide by 1000 for MHz). Mismatches indicate a configuration error at the eNB/gNB that can be resolved by the back-office team.

For NR, also verify NR_BW in service mode matches the CIQ. Common CIQ values: 20000 (20 MHz), 40000 (40 MHz), 60000 (60 MHz), 80000 (80 MHz), 90000 (90 MHz), 100000 (100 MHz).

## 2.8 MIMO, Antenna Configuration & Rank Indicator

### 2.8.1 DAS MIMO Configurations

DAS systems use dual-feed (2-port) or quad-feed (4-port) antenna architectures. PCC MIMO Configured: 2 in service mode indicates 2x2 MIMO. The theoretical throughput multiplier is: Rank 1 (SISO) = 1x, Rank 2 (2x2) = 2x, Rank 4 (4x4) = 4x. In practice, DAS environments rarely sustain Rank 4 due to antenna correlation.

### 2.8.2 NR Receive Chain Diagnostics (RX0-RX3)

Samsung service mode reports NR RX0-RX3 RSRP, revealing individual receive chain signal strength. This is critical for diagnosing antenna path issues:

- All 4 chains within 10 dB of each other: Normal. DAS dual-polarization paths are balanced.

- 2 chains strong, 2 at -140 dBm: One antenna polarization path is dead. Only Rank 2 achievable. Check DAS cabling for the affected polarization.

- Large delta (>15 dB) between RX0/RX1 and RX2/RX3: UE internal antenna imbalance OR DAS cross-polarization isolation is excessive. Verify DAS antenna type and installation.

- All chains at -140 dBm: UE is not receiving NR signal. Check NR radio unit power, NR license, and NR activation status.

### 2.8.3 NR_ANT MAX RSRP vs NR_ANT MIN RSRP

NR_ANT MAX RSRP shows the strongest receive antenna RSRP while NR_ANT MIN RSRP shows the weakest. A delta > 20 dB indicates significant antenna path imbalance. Delta > 50 dB (e.g., MAX=-44, MIN=-140) confirms at least one path is non-functional.

### 2.8.4 Troubleshooting MIMO Issues

- PCC MIMO Configured shows 1 instead of 2: Cell is configured for SISO. Verify CIQ noOfTxAntennas and noOfRxAntennas. If CIQ says 2, a back-office reconfiguration is needed.

- MIMO = 2 but throughput is at SISO levels: Check rank indicator. If RI = 1, the radio conditions don't support spatial multiplexing (antenna correlation too high, SINR too low). Improve antenna spacing or orientation.

- One or more NR RX chains at -140: Physical antenna path issue. Systematically check: DAS antenna port connectivity, splitter outputs, jumper cables, and remote unit ports.

FIX: For dead RX chains: Use a tone generator at the DAS head-end and trace the signal through each path segment to isolate the failure point. Common culprits: corroded N-type connectors at antenna, disconnected jumper at splitter, failed remote unit port.

KPI CORRELATION: MIMO issues impact: Closed Loop 2x2 MIMO Rank 2 Usage (ClosedLoop_2x2MIMORank2Usage) on LTE and NR rank counters (pmRadioTxRankDistr). Dead RX chains (NR RX at -140 dBm) produce Rank 1 only, halving DL Average UE Throughput and DL MAC Traffic Volume. Overall Closed Loop MIMO Usage below 50% warrants immediate antenna path investigation.

## 2.9 EN-DC, Carrier Aggregation & Dual Connectivity

### 2.9.1 EN-DC Architecture

EN-DC (E-UTRA NR Dual Connectivity) allows the UE to simultaneously connect to an LTE eNB (master node) and an NR gNB (secondary node). Samsung service mode confirms EN-DC via: NR_SB Status: LTE+NR. The LTE anchor provides control-plane connectivity while NR provides user-plane throughput.

### 2.9.2 LTE Anchor Impact on NR Throughput

The LTE anchor is not merely a signaling channel; it directly impacts NR throughput:

- Anchor RSRP degradation: During ENDC, the LTE anchor RSRP typically drops 10-25 dB vs. standalone due to DAS power sharing and UE TX power splitting. If anchor RSRP drops below -85, the UE may release NR and fall back to LTE-only, causing a throughput cliff.

- Anchor SINR degradation: The B66 CA leg activated during ENDC often shows 5-15 dB worse SINR than standalone. SNR below 15 on the anchor forces conservative PDCCH/PDSCH scheduling on the LTE leg.

- UPPERLAYER_IND_R15: Must show Support for ENDC to activate. If Not Supported, the UE is not receiving the NR capability indicator from the eNB.

- DCNR_RESTRICTION: Must show FALSE. If TRUE, the network is explicitly blocking EN-DC for this UE (possibly due to subscription or policy restriction).

### 2.9.3 CA-Act (Carrier Aggregation Active)

Samsung displays CA-Act status showing secondary carriers. Format: CA-Act (S1) BAND:XX BW:XXMHz. During ENDC, the LTE anchor typically activates CA with Band 66 (20 MHz) to provide additional DL capacity on the LTE leg. This B66 carrier shows its own RSRP/RSRQ/SNR values which must be monitored independently.

WARNING: CA-Act showing only S1 (one secondary carrier) when the CIQ defines multiple carriers indicates CA failure. Verify CA enablement in CIQ (LTE Carrier Aggregation / NR Carrier Aggregation sheets). Missing CA = missed throughput.

### 2.9.4 NR CA Status

NR CA Not present indicates no NR carrier aggregation is active. If the site has multiple NR carriers (e.g., n41 100 MHz + 90 MHz), these are tested independently rather than aggregated. If NR CA were enabled, both carriers would combine for peak rates exceeding 1.5 Gbps DL.

NR_Num_CC: 1 confirms single component carrier. When NR CA is active, this would show 2 or more.

### 2.9.5 Troubleshooting EN-DC / CA

- NR not activating (no 5G icon, NR fields show --): Verify UPPERLAYER_IND_R15 = Support and DCNR_RESTRICTION = FALSE. If both correct, check X2 interface between eNB and gNB. Verify NR cell is on-air and broadcasting SSB.

- EN-DC activates but drops frequently: LTE anchor is too weak. Ensure anchor RSRP stays above -80 dBm during ENDC. Optimize DAS LTE output power.

- CA not activating (CA-Not Config): Verify CA pairs are defined in CIQ (LTE Carrier Aggregation sheet). Ensure both PCells and SCells are on-air and synchronized.

FIX: To stabilize EN-DC: (1) Ensure LTE anchor RSRP > -75 dBm at all antenna-end test points, (2) Verify X2/Xn link latency < 10 ms, (3) Confirm gNB SCell activation timer is not too aggressive (SCellDeactivationTimer >= 640 ms).

KPI CORRELATION: EN-DC KPI correlation: NSA SN Addition Failure Rate (NTM_NR_ACC_03_02_096) is the primary EN-DC health KPI. NSA SN Total Drop Rate (NTM_NR_RET_03_03_001) tracks NR leg stability. FWA-specific variants (NTM_NR_ACC_03_02_149F_005, NTM_NR_RET_03_03_067_005_01) isolate fixed wireless behavior. ENDC PSCell Change Success Rates validate NR cell mobility during active EN-DC sessions.

## 2.10 Additional Service Mode Parameters

### 2.10.1 NR_SCS (Subcarrier Spacing)

Subcarrier spacing determines OFDM symbol duration and maximum supported bandwidth. 15 kHz (mu=0): Used for low-band and mid-band FDD NR (n25 at 20 MHz, n71 at 5-15 MHz). 15 kHz SCS supports up to 50 MHz channel BW per 3GPP, though deployed BWs are band-dependent. 30 kHz (mu=1): Used for sub-6 GHz mid-band NR (n41, n77, n78). Supports up to 100 MHz. 60 kHz (mu=2): Used for FR1 and FR2 overlap. 120 kHz (mu=3): Used for mmWave NR (n258, n260, n261). Supports up to 400 MHz. Verify SCS matches the CIQ and band specification. Incorrect SCS = UE cannot decode.

### 2.10.2 NR_CDRX (Connected Mode DRX)

C-DRX: Active indicates the UE is using discontinuous reception to save battery during active connections. Impact on throughput: C-DRX introduces periodic sleep cycles (typically 10-160 ms ON/OFF) during which the UE cannot receive scheduling grants. This reduces peak throughput by 5-15% and increases latency by 10-40 ms. During speed tests, the scheduler works around DRX, but captures during DRX OFF periods show momentary scheduling gaps.

- Troubleshooting high latency during speed test: If idle ping is <30 ms but loaded ping > 500 ms, C-DRX combined with bufferbloat is the cause. Not an RF issue.

FIX: To reduce C-DRX impact on throughput testing: Request back-office to temporarily disable C-DRX for the test UE IMEI (if feasible). Otherwise, note that 5-10% throughput reduction vs. theoretical is expected with C-DRX active.

### 2.10.3 NR_Tx Phy Channel Type

PUCCH (Physical Uplink Control Channel): UE is transmitting control information only (no UL data). Normal during service mode capture or when no UL data is scheduled. PUSCH (Physical Uplink Shared Channel): UE is actively transmitting uplink data. Should be observed during speed test UL phase. If PUSCH is never seen during UL speed test, the UL path may be blocked.

### 2.10.4 NR_SLICE/PDU

Shows the active network slice and PDU session. eMBBFFFFFFF = enhanced Mobile Broadband slice (default). URLLC = Ultra-Reliable Low Latency Communications. mMTC = massive Machine Type Communications. For speed testing, the UE should always be on eMBB. If a different slice is shown, throughput expectations must be adjusted for that slice QoS profile.

### 2.10.5 Paging Cycle

paging_cycle: 1280 ms indicates the DRX cycle in idle mode. Not directly relevant during active data transfer but affects call setup latency and NR reactivation time from idle.

### 2.10.6 EARFCN / NR_ARFCN

Frequency channel identifiers. Always verify against CIQ to confirm the UE is on the correct frequency. Common T-Mobile assignments: LTE: B2 EARFCN 650 (1900 MHz), B4 EARFCN 2300 (2100 MHz), B12 EARFCN 5095 (700 MHz), B66 EARFCN 66786 (2100 MHz extended), B71 EARFCN 68586 (600 MHz). NR: n25 ARFCN ~393130 (1900 MHz), n41 ARFCN ~501390/509202/521310/528144 (2500 MHz), n71 ARFCN ~123400 (600 MHz), n258 ARFCN ~2054167 (24 GHz), n260 ARFCN ~2229167 (39 GHz), n261 ARFCN ~2079167 (28 GHz).

### 2.10.7 PCI (Physical Cell Identity)

LTE PCI range: 0-503 (504 unique values). NR PCI range: 0-1007 (1008 unique values). PCI = 3 * PhysicalLayerCellIdGroup + physicalLayerSubCellId. Always verify PCI in service mode matches CIQ. PCI mismatch means the UE is on the wrong cell.

- PCI collision (two cells with same PCI within range): Causes reference signal interference, SINR degradation up to 10 dB.

- PCI confusion (two cells with same PCI detected during handover): Causes handover failure, RLF, and throughput interruption.

- PCI mod-3 collision: Two cells with PCI values having the same mod-3 result sharing the same CRS RE positions. Causes ~3 dB SINR degradation.

FIX: PCI issues require back-office replan. Use PCI planning tools to ensure no collision within 2-tier neighbor distance and no mod-3 collision between co-frequency cells.

## 2.11 TDD Configuration (n41/n77/n78)

### 2.11.1 Why TDD Matters for Throughput

NR bands n41 (2.5 GHz), n77 (3.7 GHz), and n78 (3.5 GHz) operate in Time Division Duplex (TDD) mode, meaning DL and UL share the same frequency but alternate in time. The DL-UL slot ratio directly determines the maximum fraction of air-interface time available for each direction. This is fundamentally different from FDD bands (B2, B4, n25) where DL and UL operate simultaneously on separate frequencies.

### 2.11.2 Common TDD Slot Patterns

| Pattern | Slot Config | DL % | UL % | DL Impact | UL Impact |
| --- | --- | --- | --- | --- | --- |
| DDDSU | 3DL + 1Sp + 1UL per 5ms | ~70% | ~20% | Standard | UL constrained |
| DDDDDDDSUU | 7DL + 1Sp + 2UL per 10ms | ~75% | ~20% | Optimized DL | UL constrained |
| DDSUU | 2DL + 1Sp + 2UL per 5ms | ~50% | ~40% | Reduced DL | Better UL |

Throughput implication: On TDD n41 with DDDSU pattern, the UL throughput is inherently limited to ~20-25% of what an equivalent FDD bandwidth would deliver. This is why NR n41 UL (64-79 Mbps on 90-100 MHz) is lower than NR n25 FDD UL (97.5 Mbps on only 20 MHz). The TDD UL time allocation is the binding constraint, not RF quality.

### 2.11.3 Special Slot

The Special slot (S) contains a guard period for DL-to-UL switching. The guard period duration depends on cell radius (propagation delay). In DAS environments with short cell radius (<100m), the guard period can be minimal, allowing more symbols for DL/UL in the special slot. Typical Special slot: 10 DL symbols + 2 guard + 2 UL symbols (14 total).

### 2.11.4 TDD Troubleshooting

- UL throughput much lower than expected for the bandwidth: This is normal for TDD. Calculate expected UL using the TDD UL slot ratio. For DDDSU: Max UL = FDD equivalent * 0.25 (approximately).

- DL throughput varies with time-of-day: Some networks implement dynamic TDD, adjusting DL-UL ratio based on traffic demand. Check if the gNB supports semi-static or dynamic TDD reconfiguration.

- Cross-link interference: In multi-operator DAS or sites near macro, unsynchronized TDD patterns between operators cause interference during opposite-direction slots. Verify TDD synchronization with neighboring cells.

## 2.12 RLF, RRC State & Link Stability

### 2.12.1 NR_RLF Count / NR_SA_RLF Count

Radio Link Failure (RLF) occurs when the UE loses synchronization with the serving cell. Samsung service mode reports NR_RLF Count and NR_SA_RLF Count. Any value > 0 indicates the NR link has failed and been re-established during the session. Each RLF causes a complete throughput interruption of 1-5 seconds while the UE re-establishes the connection.

Throughput impact: RLF during a speed test invalidates the result. Even one RLF mid-test can reduce measured throughput by 20-50% due to the re-establishment delay. Always check RLF count is 0 before validating speed test results.

- RLF Count > 0: Investigate serving cell signal quality (RSRP/SINR). If RF is good, check for gNB-side issues (overload, software bug, timing error). For ENDC, check LTE anchor stability.

### 2.12.2 NR_RRC State

NR_RRC: CONNECTED indicates the UE has an active RRC connection to the gNB and can receive/transmit data. NR_RRC: IDLE means no active connection (UE must perform random access before data transfer). During speed testing, always verify NR_RRC: CONNECTED. If showing IDLE during an active test, the NR leg has been released.

### 2.12.3 ASDiv (Antenna Switching Diversity)

ASDiv: NOT SUPPORTED (commonly observed) indicates the UE is not dynamically switching between antenna configurations. When supported, ASDiv can improve UL throughput by 2-5% by selecting the best UL antenna based on UL channel conditions. Absence of ASDiv is not a failure condition but represents a missed optimization opportunity.

### 2.12.4 NR-DC SCG Not Present

NR-DC (NR Dual Connectivity) uses two NR nodes simultaneously: an NR master node (MN) and an NR secondary cell group (SCG). NR-DC SCG Not present confirms the UE is not in NR-DC mode. If the site has NR-DC configured (e.g., n25 MN + n41 SCG), absence of SCG means NR-DC failed to activate. Check inter-gNB Xn interface and NR-DC configuration at both gNBs.

## 2.13 UL-Specific Throughput Factors

### 2.13.1 UL Throughput Formula

NR UL Throughput (Mbps) = N_PRB_UL * 12 * 14 * Mod_bits_UL * CR_UL * N_UL_layers * (UL_Sched%/100) * Slots_per_sec / 1e6

UL differs from DL in several critical ways: (a) UE max TX power is 23 dBm vs. gNB TX power of 33-46 dBm, making UL the weaker link. (b) UL typically uses lower modulation (64QAM max on many UEs vs. 256QAM DL) due to PA linearity constraints. (c) UL MIMO is typically 1 or 2 layers (vs. 2-4 DL layers). (d) For TDD, UL gets only 20-25% of time slots.

### 2.13.2 UL Throughput Ceiling Factors (Priority Order)

- 1. TDD UL slot ratio (for n41/n77/n78): Limits UL to ~20-25% of time resources. This is the primary UL constraint on TDD bands.

- 2. gNB UL scheduler grant allocation: The network may cap UL grants to manage interference and ensure fair sharing. Observed as consistent UL throughput regardless of RF quality.

- 3. UL bandwidth: Single carrier = single carrier BW max. UL CA disabled = UL capped at PCell BW.

- 4. UE TX Power headroom: If TX power approaches P_max (23 dBm), UL MCS must decrease. Each 3 dB power deficit costs ~1 MCS index.

- 5. UL MIMO rank: Most UEs support 1-2 UL layers. Rank 1 = 1x throughput, Rank 2 = up to 2x. DAS with 2-port antennas supports UL Rank 2.

- 6. UL modulation cap: Many UEs cap at 64QAM UL (not 256QAM), limiting per-PRB efficiency to 75% of DL.

### 2.13.3 UL CA (Uplink Carrier Aggregation)

UL CA Not Configured (commonly observed) means the UE transmits on only one carrier for UL. This is a major throughput limitation. For LTE, enabling UL CA between B2 and B66 would combine 10+20 MHz = 30 MHz UL bandwidth. For NR, UL CA is not yet widely deployed on DAS. When UL CA shows Active, verify both carriers are contributing by checking UL throughput against the combined bandwidth expectation.

## 2.15 Hardware Components: Radio Units, Baseband & Antenna

The physical hardware chain from baseband to antenna directly constrains achievable throughput. Each component has specifications that determine maximum supported bandwidth, MIMO order, output power, EVM performance, and noise figure. Understanding these specifications is essential for diagnosing throughput limitations that are hardware-imposed rather than RF-imposed.

### 2.15.1 Baseband Units

The baseband unit (BBU) performs all digital signal processing: encoding, modulation, scheduling, HARQ, and resource allocation. It is the brains of the RBS and its processing capacity determines the maximum number of simultaneously supported cells, carriers, users, and MIMO layers.

| BBU Model | Platform | Max Cells | Max BW (DL) | Technologies | Throughput Impact |
| --- | --- | --- | --- | --- | --- |
| BB 6648 | Ericsson RBS 6601 | 6-12 per BB | 100 MHz NR + LTE | LTE-A + NR SA/NSA | Adequate for 2x2 DAS |
| BB 6630 | Ericsson RBS 6601 | 6 | 60 MHz NR | LTE-A + NR | May limit at 100MHz |
| BB 6648E | Ericsson RBS 6601 | 12-18 | 200 MHz NR CA | LTE-A Pro + NR CA | Enables NR CA |

Router Platform (RP 6651): Handles transport (backhaul) connectivity. The RP determines maximum backhaul throughput. If the RP Ethernet interface is 1 GbE, total site throughput is hard-capped at ~940 Mbps regardless of RF performance. RP 6651 supports 10 GbE, removing this constraint for most DAS sites.

KEY INSIGHT: If observed DL throughput plateaus at ~940 Mbps across all carriers combined, suspect a 1 GbE backhaul bottleneck at the RP. Upgrade to 10 GbE or verify fiber backhaul capacity.

### 2.15.2 Radio Units

Radio units (RUs) perform digital-to-analog conversion, upconversion, power amplification (DL), and low-noise amplification (UL). Each radio model supports specific bands, bandwidths, and output power levels.

| Radio Model | Bands | Max BW | Max Power | MIMO | EVM Spec | NF (typ) | Throughput Impact |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Radio 4402 B25 (DC) | B2/B25/n25 | 2x20 MHz | 2x20W | 2T2R | <3.5% | 2.5 dB | Low-band workhorse |
| Radio 4402 B66 (DC) | B4/B66 | 2x20 MHz | 2x20W | 2T2R | <3.5% | 2.5 dB | AWS/PCS band support |
| Radio 6666 | n25 (NR SA) | 1x20 MHz | 1x10W | 2T4R | <4% | 3.0 dB | SA NR low-band |
| Radio 8863 B41 | n41 (NR) | 100 MHz | 4x5W | 2T2R | <4.5% | 3.5 dB | Mid-band NR primary |
| Radio 42000* | n41 | 2x100 MHz | Higher | 2T2R | <4% | 3.0 dB | Better linearity |
| Radio 38000* | n41 | 2x100 MHz | Standard | 2T2R | <4.5% | 3.5 dB | Standard unit |

* Radio 42000 and 38000 are internal radio identifiers from the CIQ radioType field; they represent different hardware revisions of the NR n41 radio platform.

### 2.15.3 Radio Unit Impact on Throughput

Output Power: Determines the maximum RSRP at the antenna end. Higher output power extends coverage radius and improves RSRP at cell edge. For DAS, the radio output is distributed across multiple antennas via splitters, so the per-antenna power equals: Radio Output (dBm) - Splitter Loss (dB) - Cable Loss (dB). If the radio output is insufficient, all downstream antennas will have weak RSRP.

EVM (Error Vector Magnitude): The radio PA (power amplifier) introduces distortion. EVM < 3.5% supports 256QAM cleanly. EVM 3.5-8% may force 64QAM on some PRBs. EVM > 8% makes 256QAM impossible, capping DL throughput at 75% of the 256QAM peak. Radio 42000 (lower EVM) vs Radio 38000 (higher EVM) explains why Sectors 1-3 may achieve slightly higher peak throughput than Sectors 4-6 on the same n41 band.

Noise Figure (NF): Determines UL sensitivity. NF of 2.5 dB (Radio 4402) means the radio adds 2.5 dB of thermal noise to the received UL signal, reducing UL SINR by that amount. Higher NF = worse UL throughput, especially at cell edge where UE TX power is already near maximum. Radio 8863 at 3.5 dB NF is 1 dB worse than Radio 4402, meaning UL throughput on n41 may be 10-15% lower than on B2/B25 for equal path loss.

Maximum Bandwidth: Radio 4402 B25 supports 2x20 MHz (dual carrier), enabling simultaneous LTE B2 + NR n25 each at 10-20 MHz. Radio 8863 B41 supports up to 100 MHz per carrier. If the radio max BW is less than the configured BW in CIQ, the cell will not activate, causing a total throughput failure for that technology.

### 2.15.4 Antenna System

DAS antenna specifications directly impact coverage pattern, MIMO performance, and per-antenna gain.

| Antenna Type | Pattern | Gain (dBi) | MIMO Support | Freq Range | Throughput Impact |
| --- | --- | --- | --- | --- | --- |
| Generic Dual Feed DAS | Omnidirectional | 2-4 dBi | 2x2 (dual-pol) | 698-2700 MHz | Standard DAS coverage |
| Generic Quad Feed DAS | Omnidirectional | 2-4 dBi | 4x4 (dual-pol x2) | 698-2700 MHz | 4x4 MIMO capable |
| Wideband Ceiling Mount | Hemi-spherical | 3-5 dBi | 2x2 | 698-6000 MHz | Covers sub-6 + CBRS |
| Directional Panel | 60-90 deg | 6-10 dBi | 2x2 | Band-specific | Higher gain, targeted |

Antenna-to-MIMO Relationship: A Generic Dual Feed DAS System (as configured in CIQ) uses two ports per antenna element, one for each polarization (+45 and -45 degrees). This supports 2x2 MIMO with Rank 2 spatial multiplexing. To achieve 4x4 MIMO, a quad-feed antenna or two co-located dual-feed antennas are required.

Frequency Range Limitation: Standard DAS antennas (698-2700 MHz) support LTE B2/B4/B66 and NR n25/n41. However, n41 at 2500 MHz is near the upper edge. Antenna gain and pattern may degrade at band edges. For mmWave NR (n258/n260/n261), entirely different antenna systems (phased arrays) are required.

Antenna VSWR: Voltage Standing Wave Ratio measures impedance mismatch. VSWR > 1.5:1 causes >4% reflected power and potential PIM. Market-specific passing thresholds: VSWR ≤ 2.1 (Return Loss > 9 dB) for RRU 4402/2203, and VSWR ≤ 1.4 (Return Loss > 16 dB) for RRU 8863. Poor VSWR reduces effective radiated power (lowers RSRP) and increases return loss (raises noise floor, lowering SINR). See Section 2.16.4 for full threshold table.

### 2.15.5 Hardware Configuration String (CIQ)

The CIQ ConfigurationType string encodes the complete hardware stack. Example: SC-93H(BB6648/RP6651)_(2x2 SM) decodes as: SC-93H = site configuration template 93H, BB6648 = baseband unit model, RP6651 = router platform model, 2x2 SM = 2x2 Spatial Multiplexing MIMO mode.

For NR: SC-593J(BB6648/RP6651 N25 - 8863)_(2x2 SM) decodes as: SC-593J = NR site configuration, BB6648/RP6651 = shared baseband, N25 = NR band n25, 8863 = Radio 8863 for n41, 2x2 SM = 2x2 MIMO.

### 2.15.6 Hardware Troubleshooting

- Radio unit fault (cell not on-air): Check radio unit alarm LEDs. Verify CPRI/eCPRI fiber link between BBU and radio. Check power supply to radio unit. A failed radio unit means zero throughput on that band/sector.

- BBU capacity exceeded: If the BBU is running more cells than rated, scheduler latency increases and throughput drops 10-30% across all cells. Check BB model capacity vs. actual cell count.

- Antenna feed mismatch: If a 4402 B25 (1900 MHz) radio is connected to an antenna port expecting 4402 B66 (2100 MHz), the DAS filters will attenuate the signal, causing 10-20 dB RSRP loss.

- Power supply degradation: Aging power supplies may not deliver rated wattage, causing radios to reduce output power. Check DC voltage at radio input vs. specification.

FIX: For mixed radio hardware (e.g., Radio 42000 on Sectors 1-3, Radio 38000 on Sectors 4-6), expect up to 10-15% throughput variance between hardware revisions. Document hardware-specific baselines for each radio model.

## 2.16 Physical Layer / Antenna System Thresholds

Market-specific passing criteria for physical layer and antenna system health. These thresholds are used during site acceptance to validate RSSI levels, branch balance, VSWR/Return Loss, and fiber integrity. All values are loaded from the Physical Thresholds Lookup sheet in DAS_Validation_Thresholds.xlsx.

### 2.16.1 RSSI Hot/Cold Thresholds

RSSI (Received Signal Strength Indicator) measures total received power including signal, interference, and noise. Hot and cold limits ensure the DAS is neither over-driving (causing PIM and saturation) nor under-powering (causing coverage holes).

| Parameter | Band | Threshold | Unit | PASS/FAIL |
| --- | --- | --- | --- | --- |
| RSSI (Hot) | PCS / AWS | > -106 | dBm | FAIL if < -106 |
| RSSI (Cold) | PCS / AWS | < -120 | dBm | FAIL if > -120 |
| RSSI (Hot) | N2500 (n25) | > -109 | dBm | FAIL if < -109 |
| RSSI (Cold) | N2500 (n25) | < -115 | dBm | FAIL if > -115 |

### 2.16.2 RSSI Target (AILG ON)

When Automatic Idle Loop Gain (AILG) is enabled, the system automatically adjusts gain to maintain RSSI within a target range. These are the expected operating values when AILG is functioning correctly.

| Band | Condition | Target RSSI (dBm) |
| --- | --- | --- |
| PCS / AWS | AILG ON | -114 to -115 |
| N2500 (n25) | AILG ON | -110 to -112 |

### 2.16.3 RSSI Imbalance (Branch Delta)

RSSI Imbalance measures the delta between Rx branch 0 and Rx branch 1 on the same RRU port. A delta exceeding 3 dB indicates an antenna system fault — typically a damaged jumper, loose connector, or failed LNA on one branch. Imbalance directly degrades MIMO Rank 2 performance and reduces DL throughput by up to 30%.

THRESHOLD: Maximum branch-to-branch RSSI delta: < 3 dB. FAIL if imbalance ≥ 3 dB on any band/sector. Check via Ericsson AMOS: get RxPowerAntenna on both branches and compute |Rx0 - Rx1|.

### 2.16.4 VSWR / Return Loss

VSWR (Voltage Standing Wave Ratio) quantifies impedance mismatch at the antenna port. Higher VSWR means more reflected power, reducing effective radiated power and potentially generating PIM (Passive Intermodulation). Thresholds vary by RRU model due to different PA designs and output impedance matching networks.

| RRU Model | Max VSWR | Min Return Loss | Throughput Impact |
| --- | --- | --- | --- |
| RRU 4402 / 2203 | ≤ 2.1 | > 9 dB | VSWR 2.1 = 11% reflected power; reduces effective RSRP by ~0.5 dB |
| RRU 8863 | ≤ 1.4 | > 16 dB | Stricter for n41 mid-band; VSWR 1.4 = 2.8% reflected power |

### 2.16.5 Fiber Loss & Rx Power

Fiber loss between the BBU and RRU (CPRI/eCPRI) must stay within tolerance. Excessive fiber loss causes bit errors on the fronthaul link, leading to increased BLER, scheduling gaps, and throughput degradation. The Rx power at the RRU SFP module must exceed -5 dBm for reliable operation.

| Parameter | Threshold | PASS/FAIL Criteria |
| --- | --- | --- |
| Fiber Loss (UL/DL) | +3 to -3 dB | FAIL if ¦loss¦ > 3 dB |
| Fiber Rx Power | RXdBm1/RXdBm2 > -5 dBm | FAIL if either Rx < -5 dBm |

KEY INSIGHT: All physical layer thresholds are loaded programmatically from the “Physical Thresholds Lookup” sheet in DAS_Validation_Thresholds.xlsx. The lookup uses parameter + band + equipment as a composite key. threshold_engine.py should never hardcode these values — always read from Excel at runtime to support market-specific threshold updates without code changes.

# 3. Speed Test Throughput Thresholds by BW & Layer

The MOP defines minimum acceptable throughput based on bandwidth and MIMO configuration. These thresholds represent the floor below which the test is considered a FAIL. All values are in Mbps.

## 3.1 SISO Configuration

| BW LTE | BW NR C1 | BW NR C2 | LTE DL | NR DL | EN-DC DL | NR-DC DL | LTE UL | NR UL | EN-DC UL | EN-DC UL SINR | UL SINR Range |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 5 | 0 | 11-20 | 10-19 | 21-39 | -- | 2.5 | 1.4 | 4-10 | 4/5/6/7/8/9/10 | 15/19/22/24/26/28/30 |
| 10 | 10 | 0 | 22-41 | 22-41 | 43-82 | -- | 5-11 | 3.9 | 8-17 | 5/15 | 12/32 |
| 20 | 20 | 0 | 43-82 | 46-87 | 82-156 | -- | 10-23 | 10-30 | 13-41 | -- | -- |

## 3.2 MIMO Configuration

MIMO configurations follow the same pattern but with approximately 2x the SISO values for 2x2, and additional scaling for higher orders. For the full MOP threshold table with all BW combinations (5/10/15/20/40/60/80/90/100 MHz) and all MIMO configurations (SISO/MIMO), refer to the DAS Validation MOP document.

KEY INSIGHT: Quick rule of thumb for expected DL: (BW in MHz) * 7.5 Mbps/MHz for LTE 2x2 256QAM, (BW in MHz) * 7.8 Mbps/MHz for NR 2x2 256QAM with 70% scheduling (NSA). For NR SA, multiply by 0.95/0.70 = 1.36x the NSA value.

# 4. Diagnostic Decision Trees

Use these structured diagnostic flows to systematically identify the root cause of throughput degradation.

## 4.1 DL Throughput Below Expected

- CHECK 1: Is the UE on the correct cell? Verify PCI and EARFCN/NR_ARFCN match CIQ.

  - NO: UE is on wrong cell. Force cell lock or relocate to dominant coverage area.

  - YES: Proceed to CHECK 2.

- CHECK 2: Is bandwidth correct? Verify BW matches CIQ.

  - NO: Back-office configuration error. Escalate for correction.

  - YES: Proceed to CHECK 3.

- CHECK 3: Is SINR > 25 dB?

  - NO: Interference or weak signal issue. Go to Section 2.2.3 for SINR troubleshooting.

  - YES: Proceed to CHECK 4.

- CHECK 4: Is RSRP within threshold (-75 to -50 for antenna end)?

  - NO: Signal strength issue. Go to Section 2.1.4 for RSRP troubleshooting.

  - YES: Proceed to CHECK 5.

- CHECK 5: Is BLER < 10%?

  - NO: High BLER. Go to Section 2.5.3 for BLER troubleshooting.

  - YES: Proceed to CHECK 6.

- CHECK 6: Is MIMO active (PCC MIMO Configured: 2)?

  - NO: SISO mode. Verify DAS antenna configuration and CIQ MIMO settings.

  - YES: Proceed to CHECK 7.

- CHECK 7: For NR NSA, is NR_DL_Scheduling > 60%?

  - NO: Scheduling bottleneck. Check EN-DC coordination, X2 interface, scheduler congestion.

  - YES: Proceed to CHECK 8.

- CHECK 8: Is CA active (if configured in CIQ)?

  - NO: CA not activating. Check SCell configuration, activation timers, frequency lock.

  - YES: If all checks pass, throughput is likely at achievable maximum for the configuration. Compare with MOP threshold table.

KPI CORRELATION: DIAGNOSTIC TREE KPI ENRICHMENT: At each CHECK step, cross-reference MS2 KPIs: CHECK 1 (correct cell) → NR Cell Availability = 100%; CHECK 3 (SINR) → PUSCH Receive Interference Power; CHECK 5 (BLER) → DL Throughput KPI trend; CHECK 6 (MIMO) → MIMO Rank 2 Usage > 60%; CHECK 7 (scheduling) → Average Connected User Number for contention; CHECK 8 (CA) → CA_Activated2CC_RATIO > 80%. This dual-validation provides audit-grade evidence for site acceptance.

## 4.2 UL Throughput Below Expected

- CHECK 1: Is UE TX Power negative?

  - NO (positive or 0): UE is power-limited. Go to Section 2.4.3 for TX power troubleshooting.

  - YES: Proceed to CHECK 2.

- CHECK 2: Is UL CA configured (UL CA Not Configured vs UL CA Active)?

  - UL CA Not Configured: UL is limited to single carrier. Maximum UL throughput is capped by single-carrier bandwidth.

  - UL CA Active: Proceed to CHECK 3.

- CHECK 3: Is NR_Tx Phy Channel Type showing PUSCH during UL test?

  - NO (showing PUCCH): UL data is not being scheduled. Verify speed test is actively uploading.

  - YES: Proceed to CHECK 4.

- CHECK 4: Are all antenna receive paths active at the gNB (check NR RX0-RX3)?

  - Dead paths (-140 dBm): UL receiver diversity is degraded. Go to Section 2.8 for antenna diagnostics.

  - All active: UL throughput is likely scheduler-limited. Compare with MOP threshold.

# 5. Speed Test Execution Best Practices

## 5.1 Service Mode - Speed Test Timestamp Rule

MOP REQUIREMENT: The timestamp between service mode screenshot and speed test result must be within a maximum interval of 3-4 minutes. Exceeding this interval invalidates the test pair because RF conditions may have changed.

- Best practice: Capture service mode, then immediately run speed test. Total elapsed time should be 30-90 seconds.

- If speed test takes longer than expected (buffering, server issues), re-capture service mode and restart.

- Verify timestamps in screenshot metadata. File naming convention YYYYMMDD_HHMMSS provides timestamp traceability.

## 5.2 Screenshot Labeling Convention

Every screenshot must be labeled with: Sector ID, Technology (L19/L21/N19/N25), Test Type (Service Mode/Speedtest), and Location (at Radio / TP1 / TP2). Example: S1 L19 at Radio, S3 N25_C1 NSA TP2.

## 5.3 TX Power Verification

To obtain a negative TX power reading, initiate a call (E911 or personal number) and capture service mode showing the negative TX power. For NR, verify both NR_Tx Pwr and ENDC Total Tx Pwr are negative.

## 5.4 NR Testing Sequence

- Step 1: Lock UE to LTE B2 (EARFCN 650). Test L19 service mode + speedtest.

- Step 2: Lock UE to LTE B4/B66 (EARFCN 2300). Test L21 service mode + speedtest.

- Step 3: Lock UE to NR n25 SA (ARFCN 393130). Test N19 service mode + speedtest.

- Step 4: Enable NR NSA. Lock to n41 C1 (ARFCN 501390/509202). Verify 5G icon. Test N25_C1 service mode + speedtest.

- Step 5: Lock to n41 C2 (ARFCN 521310/528144). Test N25_C2 service mode + speedtest.

- Step 6: Enable NR All Float. Test LTE All / NR All combined speedtest.

## 5.5 Common Pitfalls

- Speed test server selection: Use a nearby server (same metro). Distant servers add 50-200 ms latency and 10-30% throughput reduction.

- Background data: Disable all background apps, cloud sync, and OS updates before testing. These consume bandwidth and skew results.

- UE thermal throttling: After 10+ consecutive speed tests, the UE modem may thermal-throttle, reducing throughput by 10-30%. Allow 2-3 minute cool-down between tests.

- Battery level: Below 20% battery, some UEs reduce modem power to conserve energy. Keep UE charged above 30%.

- CDRX impact: C-DRX can reduce measured throughput by 5-10%. Note C-DRX state in documentation.

# 6. Essential Formulas & Calculations

## 6.1 Theoretical Peak Throughput

LTE DL Peak = N_PRB * 12 * 14 * Mod_bits * CR * N_layers * (1-OH%) * 1000 / TTI_ms [Mbps]

NR DL Peak = N_PRB * 12 * N_symbols_per_slot * Mod_bits * CR * N_layers * (1-OH%) * slots_per_subframe / 1e6 [Mbps]

## 6.2 Throughput Efficiency

Throughput Efficiency % = (Measured DL / Theoretical Peak DL) * 100

Per-MHz Efficiency = Measured DL (Mbps) / Bandwidth (MHz) [Mbps/MHz]

Expected per-MHz efficiency: LTE 2x2 256QAM = 5-9.5 Mbps/MHz. NR 2x2 256QAM = 6-8 Mbps/MHz (NSA), 7-10 Mbps/MHz (SA).

## 6.3 UL Throughput Estimation

NR UL Peak (TDD) = N_PRB * 12 * 14 * UL_Mod_bits * UL_CR * UL_layers * (UL_slot_ratio) * slots_per_sec / 1e6

Example for 100 MHz n41 UL (30kHz, DDDSU): 273 PRBs * 12 * 14 * 6 (64QAM) * 0.93 (MCS Table 1, Index 19) * 1 layer * 0.20 (UL slots) * 2000 / 1e6 = 102 Mbps theoretical. Observed 77-79 Mbps = 76% of theoretical, with gap from scheduling overhead and control channels.

## 6.4 ENDC Combined Throughput

ENDC DL Total = LTE PCell DL + LTE SCell(s) DL + NR SgNB DL

ENDC UL Total = LTE PCell UL + NR SgNB UL (if split bearer) OR max(LTE UL, NR UL) (if switched bearer)

## 6.5 Path Loss Budget

Max Allowable Path Loss = eNB TX Power (dBm) + Antenna Gain (dBi) - Cable Loss (dB) - Required RSRP_min (dBm)

Measured Path Loss = Head-end output (dBm) - Feeder loss - Splitter loss - Jumper loss + Antenna gain - Measured RSRP (dBm)

Note: Antenna gain ADDS to effective radiated power (positive term). Cable/splitter/jumper losses SUBTRACT (negative terms). RSRP is the result after all gains and losses.

## 6.6 SINR Estimation from RSRP

SINR_est (dB) = RSRP_serving - 10*log10(sum(10^(RSRP_interferer_i/10)) + N_thermal) [dB]

N_thermal = -174 + 10*log10(BW_Hz) + NF [dBm]   (e.g., -174 + 80 + 5 = -89 dBm for 100 MHz)

2.15 NR-DC (NR Dual Connectivity — 5G SA Architecture)

FIX: To validate 256QAM UL: test at radio end (short cable run), ensure TX power is < -5 dBm, monitor UL throughput for results exceeding the 64QAM ceiling (>350 Mbps on 100 MHz NR). Persistent UL throughput cap at 64QAM levels despite high UE transmit power indicates the DAS UL noise floor is preventing MCS advancement.

TDD slot ratio: On n41/n77/n78 TDD bands, only ~20-25% of slots are UL. UL 256QAM improves per-PRB efficiency but cannot change the TDD UL ratio. Combined effect: 256QAM UL on 100 MHz n41 with DDDSU pattern ≈ 450-500 Mbps theoretical UL peak.

UL SINR at the gNB: Must exceed 28 dB consistently. PIM, intermod, or co-channel interference from other DAS sectors degrades UL SINR directly.

DAS noise figure: Cascaded NF must be <6 dB. High NF raises the gNB receiver noise floor, preventing 256QAM UL scheduling even when the UE has strong signal.

UL 256QAM activation in DAS requires careful noise management. The UE's signal must arrive at the DAS remote unit (or BDA) with sufficient UL SNR. Key factors:

2.15.1 Definition & Architecture

2.15.2 Throughput Benchmarks

NR-DC in Samsung service mode: NR_SB Status will show NR+NR (or the secondary NR PCI/ARFCN will be distinct from the primary). ENDC Total Tx Pwr field will still show combined UL TX power.

NR-DC (Release 15):  NR gNB (MN) + NR gNB (SN) → 5GC only. Both nodes are 5G NR. Enables mid-band + mmWave aggregation on pure SA networks.

EN-DC (Release 15):  LTE eNB (MN) + NR gNB (SN) → EPC or 5GC. Requires LTE anchor. Most deployed mode today.

Architecture comparison:

NR-DC (New Radio Dual Connectivity) is a 3GPP Release 15 feature that allows a 5G SA UE to simultaneously connect to two 5G gNBs — a Master Node (MN) and a Secondary Node (SN) — both over the 5G Standalone core (5GC). Unlike EN-DC which anchors to an LTE master, NR-DC operates entirely within the 5G NR/5GC domain.

2.15.3 NR-DC vs EN-DC: Key Differences for DAS Validation

NR-DC throughput improvement vs single connectivity (academic simulation): +14.07% throughput, -32.94% latency, -11.11% packet loss.

5G Advanced 6-CA: Samsung Galaxy S25 (X80) achieved 4.3 Gbps DL on 6-carrier aggregation (low-band + mid-band only).

UL peak: 2.2 Gbps — 60% of mmWave radio resources allocated for UL (vs. typical 20%). World record as of Nov 2024.

DL peak: 4.95 Gbps — aggregating 100 MHz n41 (2.5 GHz) + 800 MHz n260 (39 GHz mmWave)

T-Mobile NR-DC field records (2024, Ericsson + Qualcomm X80, SoFi Stadium, Southern California):

2.15.4 Troubleshooting NR-DC

DAS implication: For NR-DC mmWave DAS, the mmWave remote unit must be connected to a 5G SA CU/DU (not a legacy LTE-anchored BBU). This requires O-RAN or CPRI fronthaul capable of handling both mid-band and mmWave gNB functions simultaneously.

UL resource allocation: NR-DC enables flexible split of UL resources between MN and SN bands. The mmWave SN can be allocated more UL time than in TDD patterns typical for eMBB (up to 60% UL allocation tested in field vs. default ~20%).

NR-DC requires 5G SA core (5GC). Venues with non-SA T-Mobile DAS deployment cannot use NR-DC; they use EN-DC. SA deployment is required as a prerequisite.

EN-DC requires LTE anchor cell active. If LTE RSRP degrades, NR secondary drops. NR-DC does not have this LTE dependency — the NR MN directly manages the SN.

Dimension

FIX: For DAS validation, NR-DC throughput tests require SA-capable infrastructure, a supported UE (X80 or equivalent), and verified mmWave coverage at the test point. Use NR-DC specifically when testing mmWave DAS integration into SA network.

Step 4: Check NR-DC CA combination is in T-Mobile's allowed combination list for the UE's capability. Unsupported band combinations are rejected by RRC.

Step 3: Verify UE capability. Not all devices support NR-DC. Qualcomm X80 (2024 flagship devices) supports NR-DC; X65 and earlier typically support EN-DC only.

Step 2: Check that the secondary NR cell (e.g., mmWave n260) has sufficient RSRP at the test location. FR2 (mmWave) has narrow beam coverage — confirm beam is pointing correctly.

Step 1: Verify 5G SA network is active (NR_RRC State = RRC_CONNECTED, no LTE anchor). If LTE anchor is active, the mode is EN-DC, not NR-DC.

NR-DC not activating (only one NR cell visible in service mode):

2.16 256QAM Uplink (UL High-Order Modulation)

WARNING: Do not mistake a RedCap UE's 20 MHz bandwidth limitation for a DAS bandwidth fault. Always confirm the test UE's modem type (X75/X80 preferred) and verify NR_BW = full carrier bandwidth before interpreting throughput results.

FIX: If UL/DL throughput is unexpectedly capped and RF parameters look clean, check gNB scheduler utilization. High PDCCH utilization from many low-bandwidth devices (RedCap/IoT) can limit scheduling grants for the eMBB speed test UE.

MOP test recommendation: When possible, conduct speed tests outside of peak IoT device load periods, or use a dedicated speed-test slice (5QI=9) separate from IoT QoS flows.

PRB contention: RedCap UEs consuming PRBs from the shared DL scheduler reduce available PRBs for the speed-test UE, potentially capping observed throughput below the theoretical max.

RedCap devices are not used for MOP speed tests (they cannot reach the minimum throughput thresholds on their own). However, in a venue where RedCap devices are present alongside standard UEs:

2.17 RedCap / NR-Light (3GPP Release 17 Reduced Capability)

2.17.1 Definition & Purpose

2.16.1 Definition & 3GPP Specification

2.16.2 Device Support Status (2024)

Practical constraint: 256QAM UL requires sustained SINR >28 dB at the gNB receiver (uplink). This is achievable at short range (DAS radio end) but often not at antenna end where path loss is higher.

256QAM UL:  8 bits/symbol. At 100 MHz NR, SISO, theoretical max ~600 Mbps UL — 33% improvement over 64QAM ceiling.

64QAM UL:   6 bits/symbol. At 100 MHz NR, SISO, max ~450 Mbps UL.

UL 256QAM throughput gain vs 64QAM:

256QAM Uplink (UL 256QAM) is defined in 3GPP TS 38.214 (NR Physical Layer Procedures for Data), enabling the PUSCH to use 256-QAM modulation with up to 8 bits per symbol. This is specified in MCS Table 2 (Table 5.1.3.1-2) for PDSCH and the corresponding UL MCS table for 256QAM PUSCH. It was introduced in 3GPP Release 15 as optional UE capability and is network-configurable via RRC signaling (mcs-Table-PUSCH-r17).

2.16.3 Identifying 256QAM UL in Samsung Service Mode

Samsung Galaxy S25 series (X80): Confirmed 256QAM UL support per T-Mobile 5G Advanced tests (4.3 Gbps DL, commensurate UL improvement).

MediaTek Dimensity 9300 and 9400 (2024): Supports UL 256QAM per 3GPP Rel-16+ capability.

Qualcomm Snapdragon X80 (Snapdragon 8 Gen 4 devices, 2024+): 3.5 Gbps UL peak capability. Supports 256QAM UL with 6x CA and NR-DC. First confirmed 5G-Advanced-ready modem commercially deployed.

Qualcomm Snapdragon X75 (Snapdragon 8 Gen 2 devices, 2023): 256QAM UL support per spec capability but network-dependent. T-Mobile network activation status varies by market.

2.16.4 DAS Implications for UL 256QAM

For confirmation: use UE modem diagnostic logs (QXDM) or gNB KPI counters (PUSCH MCS histogram) to verify 256QAM UL scheduling

UL throughput > 350 Mbps on 100 MHz NR in SISO: this exceeds 64QAM ceiling and confirms 256QAM or 4-layer UL MIMO is active

NR_Tx Pwr well below max (< +5 dBm, ideally negative) — 256QAM UL requires good SNR at gNB, which typically means the UE is close to the antenna

NR_Tx Phy Channel Type = PUSCH (confirming data is being transmitted, not just control)

Samsung service mode does not display UL MCS index directly. Indicators of 256QAM UL activity:

2.17.2 eRedCap (Release 18 Enhancement)

Requires: 5G SA core (5GC) — cannot operate on NSA/EN-DC architecture

Peak DL: ~226 Mbps | Peak UL: ~120 Mbps

Carrier aggregation: NOT supported in Rel-17 RedCap

DL MIMO layers: 1 (with 1 Rx branch) or 2 (with 2 Rx branches)

Receive antenna branches: 1 or 2 (vs 4 for standard NR eMBB UE)

Maximum bandwidth: 20 MHz (FR1) / 100 MHz (FR2) — vs 100 MHz+ for standard NR

RedCap key constraints vs standard NR UE:

5G Reduced Capability (RedCap), standardized in 3GPP Release 17 (completion mid-2022), defines a 5G NR device profile specifically for IoT applications that do not require the full capabilities of eMBB 5G. Target use cases: wearables (smartwatches, medical monitors), industrial wireless sensors, and video surveillance cameras.

2.17.3 DAS Implications

3GPP Release 18 (5G Advanced) introduced eRedCap for even simpler devices: max 5 MHz bandwidth (FR1), peak data rate capped at 10 Mbps both DL and UL. Targets price-sensitive sensors and tracking devices where LTE-M/NB-IoT cannot meet latency or throughput needs. Commercial eRedCap modules began emerging in 2024.

2.17.4 Speed Test Impact

High-density venues (stadiums, airports): In mass-gathering events, a large influx of RedCap wearables (e.g., medical monitors, smart badges) could saturate the PDCCH scheduling capacity if not properly managed with RedCap-specific scheduling policies defined in 3GPP TS 38.300.

SA Core requirement (critical constraint): RedCap operates only on 5G SA networks. DAS venues still running EN-DC (LTE+NR NSA) architecture cannot serve RedCap devices on NR. This is a primary deployment constraint for venues operating T-Mobile's pre-SA DAS infrastructure.

BWP (Bandwidth Part) management: In mixed RedCap + eMBB deployments, the gNB assigns RedCap UEs a dedicated BWP (≤20 MHz) while eMBB UEs use the full carrier bandwidth. This prevents RedCap devices from monopolizing PRBs on narrow-band configurations. DAS operators must ensure the baseband/CU is configured to support BWP partitioning.

SISO/single-antenna alignment: Legacy passive DAS (SISO, 1T1R) naturally aligns with RedCap's 1-Rx-branch profile. No MIMO infrastructure upgrade is needed to serve RedCap devices. However, this also means full-capability UEs will not benefit from MIMO on legacy DAS.

RedCap device behavior in DAS-covered venues has three key implications:

2.18 FR2 / mmWave DAS Validation (n258 / n260 / n261)

mmWave bands (FR2) represent a fundamentally different propagation environment from sub-6 GHz FR1 bands. Validation methodology, RSRP thresholds, and throughput expectations all differ significantly. This section covers T-Mobile mmWave bands: n261 (27.5–28.35 GHz), n258 (24.25–27.5 GHz), and n260 (37–40 GHz).

2.18.1 Definition & Frequency Characteristics

FR2 (mmWave) key characteristics vs FR1 (sub-6 GHz):

- Path loss: 20–30 dB higher per decade than FR1 at equivalent distance. Friis equation: PL = 20·log10(4πd·f/c). At 28 GHz vs 2.5 GHz: ~21 dB additional free-space loss.

- Cell radius: Practical indoor mmWave cells are 10–23 meters. NLoS penetration loss: 20–40 dB through drywall, concrete blocks entirely.

- Channel bandwidth: 50, 100, 200, or 400 MHz supported (per 3GPP TS 38.101-2). T-Mobile n261 deployment: typically 400 MHz BW for maximum throughput.

- SSB beams: FR2 uses up to 64 SS/PBCH blocks (SSBs) with narrow beams (L1-RSRP per beam).

- Subcarrier spacing: 120 kHz (µ=3) or 240 kHz (µ=4) for FR2 (vs 15/30/60 kHz for FR1).

- TDD only: All mmWave bands are TDD. DL/UL ratio same considerations as n41/n77/n78.

2.18.2 RSRP Thresholds (FR2-Specific)

mmWave RSRP thresholds differ substantially from FR1 because of higher path loss and smaller cell size. Do NOT apply FR1 RSRP thresholds to FR2 measurements.

mmWave DAS RSRP Thresholds (n258/n260/n261):

> -70 dBm  : Excellent. Full throughput. 256QAM DL achievable. LoS or near-LoS.

-70 to -85 dBm: Good. High throughput. Typical indoor coverage target.

-85 to -100 dBm: Marginal. Throughput degraded. MCS reduced. Edge of practical coverage.

< -100 dBm : Poor. Beam management failure likely. Handover to FR1 expected.

Note: Minimum indoor NLoS target for mmWave DAS is RSRP > -100 dBm (per industry benchmarks for industrial indoor scenarios). Compare to FR1 minimum of -110 to -115 dBm.

2.18.3 Throughput Benchmarks & Validation Targets

Expected throughput for mmWave DAS (n261, 400 MHz BW, SISO, good RSRP):

LoS / near-LoS (RSRP > -70 dBm):    DL 400–560 Mbps | UL 50–70 Mbps (400 MHz BW, SISO)

Limited LoS (RSRP -70 to -90 dBm):  DL 200–425 Mbps | UL 20–40 Mbps

Marginal (RSRP -90 to -100 dBm):    DL <150 Mbps | UL <15 Mbps

Theoretical peak (400 MHz, 4-layer MIMO): DL >4 Gbps, UL >1 Gbps in ideal conditions.

MOP validation note: mmWave DAS sites should be tested in static position (no movement). Beam tracking during movement can cause 100–500 ms interruptions.

2.18.4 DAS Vendors Supporting mmWave

Not all DAS vendors support FR2. Confirmed mmWave-capable DAS platforms (as of 2024–2025):

- Ericsson: Radio 4480 / 6488 (mmWave remote units), supports n258/n261/n260 with eCPRI fronthaul.

- CommScope ION-E: mmWave extension module (28 GHz) for stadium/venue deployments.

- Corning ONE Wireless: mmWave distribution module for n261 deployments.

- JMA TEKO / NCHA: mmWave variant available for select venue deployments.

- Small cell alternative: For many indoor mmWave deployments, pRRUs or small cells (e.g., Ericsson Air 6419, Nokia FastMile mmWave) replace distributed DAS architecture due to extreme path loss making distribution network insertion loss prohibitive.

2.18.5 Samsung Service Mode for mmWave

Enabling mmWave in Samsung service mode: *#0011# → UE SETTING & INFO → SETTING → PROTOCOL → NR5G → Sub6/mmWave → ALL ENABLE (enables both FR1 and FR2 simultaneously).

Key mmWave parameters to monitor:

- NR_BAND: Will show mmWave band number (261, 258, or 260) when camped on FR2.

- NR_ARFCN: Will show FR2 ARFCN values (2054166–3279165 for n258; 2270833 for n261 center).

- NR_BW: Confirm 400 MHz (or 100/200 MHz per site design).

- RSRP: Use FR2 thresholds above (not FR1 thresholds).

- L1-RSRP (beam-level): In FR2, L1-RSRP per SSB beam is critical. UE should show best-beam RSRP in NR5G service mode fields.

2.18.6 Troubleshooting mmWave DAS

SYMPTOM: UE does not connect to mmWave (stays on FR1):

→ Verify mmWave enabled in service mode (ALL ENABLE setting)

→ Confirm n261/n258 band is enabled in network operator settings

→ Check RSRP > -100 dBm at test location; mmWave will not camp below this floor

→ Verify gNB is broadcasting FR2 SSBs (check NR_ARFCN in service mode)

SYMPTOM: mmWave connects but throughput is low (<100 Mbps on 400 MHz):

→ RSRP marginal (-85 to -100 dBm): reposition to LoS or closer to antenna

→ Beam management issue: check for rapid NR_ARFCN changes (beam hopping = instability)

→ DAS insertion loss too high: verify cascaded loss budget <20 dB for 28 GHz

→ Connector/cable VSWR issue: at 28 GHz, 0.5 dB connector loss has high impact

SYMPTOM: Speed test result interrupted or shows 0 Mbps mid-test:

→ Beam tracking failure during test (stay static during mmWave testing)

→ UE switched from FR2 to FR1 mid-test (FR2 RSRP dropped below camping threshold)

2.19 L1/L2 Triggered Mobility (LTM) — 3GPP Release 18

L1/L2 Triggered Mobility (LTM) is a 5G Advanced feature introduced in 3GPP Release 18 (finalized December 2023) that enables faster cell switching using lower-layer signaling rather than traditional RRC (Layer 3) procedures.

2.19.1 Definition & Mechanism

LTM reduces handover interruption time by pre-configuring candidate target cells via RRC, then triggering the actual switch using MAC Control Element (MAC CE) signaling based on real-time L1 measurements (L1-RSRP).

LTM interruption times vs traditional handover:

LTM (Rel-18):              20–30 ms interruption

Traditional Xn handover:   50–90 ms interruption

N2-based handover:         100–300 ms interruption

LTM mechanism steps:

1. gNB pre-configures candidate cells via RRC reconfiguration (before trigger)

2. UE measures L1-RSRP for SSB beams of candidate cells continuously

3. When serving cell quality drops below threshold, MAC CE triggers cell switch

4. UE pre-synchronizes DL/UL with target cell before switch completes

5. Random access to target cell may be contention-free (no full RACH needed)

2.19.2 Release Scope & Evolution

- Release 18 LTM: Intra-gNB only (candidate cells served by same basestation).

- Release 19 roadmap: Inter-gNB LTM via Xn signaling with data forwarding.

- Relationship to CHO (Conditional Handover, Rel-16): CHO uses L3 triggering; LTM uses L1/L2 for faster execution.

NOTE: Some industry references (including Ericsson blog Aug 2024) discuss LTM under Rel-17/18. Per 3GPP TS 38.300 and IEEE publication 10744020, LTM was formally specified in Release 18.

2.19.3 DAS Speed Test Impact

LTM impact on speed tests is minimal in well-designed DAS environments:

- The 20–30 ms interruption is shorter than most speed test packet retransmission timers.

- In DAS sites, LTM would occur between sectors (PCIs) on the same headend gNB — this is the intra-gNB scenario covered by Rel-18 LTM.

- If a speed test shows a brief throughput spike followed by a short gap, check for PCI change in Samsung service mode (NR_PCI field changing mid-test).

- DAS environments with many sectors and overlapping coverage benefit most from LTM as the UE can smoothly transition between sectors without throughput loss.

2.19.4 Troubleshooting LTM Events

SYMPTOM: Intermittent throughput drops of 20–100 ms during speed test:

→ Check NR_PCI for mid-test changes (sector handover occurring)

→ Verify NR_RLF count is 0 (LTM should not increment RLF counter)

→ If LTM is the cause, no action needed — this is expected behavior on multi-sector DAS

→ If RLF count increments: this is NOT LTM, it is a Radio Link Failure — investigate RSRP and SINR coverage gap between sectors

2.20 BWP Adaptation & Advanced UE Power Saving

3GPP Release 15 introduced Bandwidth Parts (BWP) to enable UE power saving by operating on a narrower bandwidth than the full channel bandwidth. Release 17 added advanced power saving enhancements. These features can significantly reduce speed test throughput if triggered by network configuration during testing.

2.20.1 BWP (Bandwidth Part) Adaptation

Definition: A BWP is a contiguous set of PRBs within the channel bandwidth. A UE can be configured with up to 4 DL BWPs and 4 UL BWPs. Only one BWP is active at a time per direction.

Power saving mechanism: gNB configures a narrow "default BWP" (e.g., 20 MHz within a 100 MHz channel). UE switches to narrow BWP after the BWP Inactivity Timer expires during low-traffic periods. Speed tests immediately trigger switch to the wider "active BWP" upon data arrival.

Speed test throughput impact:

- If BWP switch delay occurs at test START: first 2–5 seconds may show reduced throughput

- BWP switch delay: typically 1–5 ms (network-configured)

- After switch to full-BW BWP: throughput returns to full expected value

- Samsung service mode: NR_BW field reflects the active BWP bandwidth, not the channel bandwidth.

IMPORTANT: If NR_BW shows 20 MHz on a 100 MHz channel site, the UE is on the default narrow BWP. Wait 2–3 seconds after starting data transfer and re-check NR_BW — it should switch to 100 MHz.

2.20.2 MIMO Layer Reduction (Dynamic Antenna Adaptation)

Definition: The network can configure different maximum MIMO rank (Lmax) per BWP. Under low-traffic, the gNB may instruct the UE to reduce active receive chains.

Throughput impact: Direct proportional reduction.

4-layer MIMO → 2-layer: ~50% throughput reduction

4-layer MIMO → 1-layer (SISO): ~75% throughput reduction

DAS implication: DAS sites typically deliver 1–2 MIMO layers. If gNB is configured with MIMO layer reduction for power saving, a DAS site may be artificially capped below its architectural capability. Escalate if NR RI (Rank Indicator) is consistently below expected for that DAS antenna configuration.

2.20.3 Cross-Slot Scheduling (K0 Offset)

Definition: K0 is the slot offset between PDCCH (control) and PDSCH (data) scheduling. K0=0: data in same slot (standard). K0>0: data in future slot (allows UE micro-sleep).

Speed test impact: Minimal direct throughput impact. Increases latency slightly (by K0 × slot duration). With 30 kHz SCS and K0=4: adds ~0.14 ms latency per slot. Cross-slot scheduling is primarily a power-saving feature, not a throughput-limiting one.

2.20.4 Troubleshooting Power Saving Issues

SYMPTOM: First 3–5 seconds of speed test are slow, then throughput jumps to normal:

→ BWP switching delay: normal behavior. Conduct test over 15–30 seconds, discard first 5 seconds.

SYMPTOM: NR_BW shows narrow bandwidth (e.g., 20 MHz) but site CIQ shows 100 MHz channel:

→ UE is on default narrow BWP. Start a data transfer and observe NR_BW field — it should switch to full BW within 1–2 seconds as gNB activates the wide BWP.

SYMPTOM: Throughput consistently below SISO baseline despite good RSRP and SINR:

→ Check NR_BW (BWP may be stuck narrow due to misconfiguration)

→ Check RI / rank (MIMO layer reduction may be active)

→ Verify TDD slot config (UL/DL ratio affects both DL and UL caps — see section 2.11)

→ Check NR_CDRX status (C-DRX cycles may be causing periodic sleep — see section 2.10.2)

# 7. DAS Vendor-Specific Throughput Analysis

This section provides vendor-specific throughput baselines, common failure patterns, and step-by-step troubleshooting decision trees for the five DAS vendors used in integration exercises. The post-check data from Ericsson AMOS reflects the RAN-to-DAS headend interface: CPRI status is the BBU-to-RRU link (before the DAS), VSWR at the RRU port includes DAS headend input impedance, and elevated RSSI in DAS environments often indicates PIM from the distribution network.

KEY: The post-check sees the Ericsson RAN side only. DAS distribution issues (headend-to-remote) manifest indirectly as RSSI elevation (PIM), VSWR anomalies (headend input impedance), or throughput degradation without clear RAN-side alarms.

## 7.1 DAS Throughput Baselines (All Vendors)

The following throughput ranges represent expected per-sector, per-band performance measured at the DAS remote unit output. Actual UE throughput will be lower due to path loss, scheduling, and loading. Degradation below 80% of baseline minimum warrants investigation; below 50% indicates a critical DAS or RAN issue.

7.1.1 LTE Throughput Baselines

Band 5 (850 MHz, 10 MHz BW): DL 15-30 Mbps | UL 5-10 Mbps. Coverage layer, not capacity. Expect lower modulation (16QAM-64QAM) in noise-limited environments.

Band 66 / AWS (1700/2100 MHz, 20 MHz BW): DL 40-75 Mbps | UL 10-25 Mbps. Primary LTE capacity layer. 256QAM achievable with SINR >20 dB.

7.1.2 NR Throughput Baselines

n77 / C-Band (3.7-3.98 GHz, 40-100 MHz BW): DL 200-400 Mbps | UL 30-60 Mbps. Massive throughput increase with NR. 4x4 MIMO + 256QAM at good SINR.

n2 (1900 MHz, 15-20 MHz BW): DL 50-100 Mbps | UL 15-30 Mbps. DSS (Dynamic Spectrum Sharing) with LTE may reduce effective NR throughput by 30-50%.

7.1.3 Vendor Throughput Variations

CommScope ERA and JMA TEKO achieve the highest end of baseline ranges due to mature digital DAS processing. ADRF SDR/ADX may show 5-10% lower throughput ceiling due to processing latency (see Section 7.3). Corning Everon and SOLiD ALLIANCE perform within standard baseline ranges. Legacy platforms (CommScope FlexWave, Corning ONE, ADRF SDR) may have reduced NR n77 support or bandwidth limitations.

## 7.2 CommScope (ERA, ION-E, FlexWave)

CommScope is the most deployed DAS vendor in large venues (stadiums, airports, convention centers). The ERA platform is the current-generation digital DAS supporting up to 8 bands and 48 remote units over single-mode fiber.

7.2.1 Common Failure Patterns

Headend Input Overload: When NR C-Band is added to an existing LTE-only ERA DAS, the composite input power from Ericsson RRUs may exceed the headend input threshold. Symptom: composite power alarm on headend, all sectors degrade simultaneously. Fix: attenuate RRU output power or reconfigure headend input gains. Verify headend firmware supports NR bandwidth.

PIM from Distribution Connectors: Elevated RSSI on Ericsson RRU with intermittent throughput drops. PIM from DAS distribution components (connectors, splitters, combiners, antennas). Worse with multi-band operation. Fix: PIM sweep from RRU output through DAS chain. Torque-check all DIN 7/16 and 4.3-10 connectors. Common culprits: combiner/splitter junctions.

Remote Unit Fiber Fault: Single remote unit offline while others on same headend are OK. Fiber break or excessive loss on distribution fiber. Fix: OTDR on DAS distribution fiber, check patch panel connections, clean fiber connectors (LC/SC type).

Firmware Mismatch: Intermittent remote unit dropouts, CRC errors on management link. Headend and remotes running different firmware. ERA requires headend upgrade before remote units.

Gain Imbalance: Uneven coverage within venue. Remote units show different signal levels on same sector. Fix: review headend gain tables, verify cable loss compensation matches actual installed lengths.

7.2.2 CommScope Troubleshooting: No Throughput

Step 1: Check Ericsson CPRI status (AMOS: get RiLink=* operationalState)

Step 2: If CPRI OK, check RRU RF port VSWR (AMOS: get AntennaNearUnit=* vswrFault)

Step 3: If VSWR OK, check CommScope headend alarms via ERA Manager/NMS

Step 4: If headend OK, check DAS distribution fiber to affected remote unit

Step 5: If fiber OK, check remote unit power and LED status

Step 6: If remote unit OK, check passive antenna connection at remote unit output

7.2.3 CommScope Troubleshooting: Low Throughput

Step 1: Verify Ericsson cell is enabled and not in degraded mode

Step 2: Check RSSI on RRU - if elevated (>-106 dBm for PCS/AWS, >-109 dBm for N2500), suspect PIM from DAS

Step 3: Check headend input power levels - should match RRU output config

Step 4: Check headend gain settings per band/sector

Step 5: Verify remote unit output power matches design spec

Step 6: Check for interference from co-located DAS bands (guard band spacing)

Step 7: Measure actual path loss from remote unit to UE position

## 7.3 JMA Wireless (TEKO, NCHA)

JMA Wireless is strong in stadiums and large campus deployments. The TEKO platform supports single-fiber BiDi (Bi-Directional) operation, reducing fiber count by 50%.

7.3.1 Common Failure Patterns

BiDi Fiber SFP Pairing Error: TEKO Remote Point shows offline despite fiber being connected. Single-fiber BiDi uses 1310nm TX / 1490nm RX on one end and the reverse on the other. If same-wavelength SFPs are installed on both ends, the link loops instead of connecting. Fix: verify BiDi SFP pairing - Hub and TRP must have OPPOSITE wavelength SFPs (Type A <-> Type B). Check color coding on SFP labels.

Hub DSP Overload: Hub shows DSP utilization alarm. Some bands drop. Adding new NR carrier fails. NR with 4x4 MIMO uses significantly more DSP than LTE. Fix: review hub DSP allocation in TEKO Manager. Options: reduce MIMO layers, add second hub, or upgrade DSP card.

TRP Power Supply Degradation: TRP goes offline intermittently during peak load. NR C-Band TRPs draw more power. Fix: check TRP power supply voltage/current draw. Verify PoE switch capacity if applicable.

Gain Calibration Drift: Gradual performance degradation over weeks. Hub auto-calibration not running or failing. Fix: run manual calibration from TEKO Manager. If it fails, check fiber degradation on calibration path.

7.3.2 JMA Troubleshooting: No Throughput

Step 1: Check Ericsson CPRI link to RRU feeding the TEKO hub

Step 2: If CPRI OK, log into TEKO Manager and check hub status

Step 3: Check hub-to-TRP fiber link status in TEKO Manager

Step 4: If BiDi fiber, verify SFP pairing (Type A/Type B at opposite ends)

Step 5: Check TRP power LED and management connectivity

Step 6: If TRP powered but no RF, check TRP RF output with spectrum analyzer

## 7.4 ADRF (SDR, ADX)

ADRF is common in mid-size venues (hotels, hospitals, offices). Known for competitive pricing but has specific latency and thermal considerations.

CRITICAL: ADRF digital processing adds measurable latency. If one-way delay through the DAS path exceeds 10ms, throughput will be impacted due to HARQ timing budget constraints. Always measure DAS latency as part of speed test validation.

7.4.1 Common Failure Patterns

Latency-Induced Throughput Loss: Throughput lower than expected despite good signal levels. One-way DAS latency >10ms. Older SDR firmware has higher latency per hop. Daisy-chained remote units compound it. Fix: upgrade to latest firmware (reduces latency 30-40%), reduce daisy-chain depth (max 3 hops recommended), verify HARQ timing budget.

Band Interaction Spurs: Spurious signals near DAS output, intermittent throughput drops on specific band combinations. Most common with B5+B66+n77 triple-band. Fix: check ADRF filter configuration, adjust per-band power levels to reduce IM products.

Remote Unit Overheating: Thermal alarm, unit powers down during peak hours. Ceiling-mounted units with restricted airflow most affected. NR-enabled remotes draw more power. Fix: improve ventilation, reduce TX power if possible, install heat sinks.

Config Sync Failure: Hub shows config changes but remotes don't reflect them. Fails silently on some firmware versions. Fix: reboot affected remote units after config changes. Update firmware.

7.4.2 ADRF Troubleshooting: Low Throughput

Step 1: Measure actual latency through DAS path (one-way delay)

Step 2: If latency >10ms, check daisy-chain depth and firmware version

Step 3: Check per-band output power at remote unit with power meter

Step 4: Run spectrum analyzer at remote unit output to check for spurs

Step 5: Verify NR-specific configuration (bandwidth, MIMO mode)

Step 6: Compare hub gain settings against original design document

## 7.5 Corning (Everon, ONE, SpiderCloud)

Corning is strong in healthcare and hospitality venues. The Everon platform is current-generation. The older ONE platform has limitations with NR support.

WARNING: Corning ONE platform may NOT support NR n77 (C-Band). The analog front-end filters were designed pre-5G. Always verify ONE model and revision against Corning NR compatibility matrix before attempting NR integration.

7.5.1 Common Failure Patterns

Remote Unit Capacity Limits: Throughput degrades with multiple UEs but no DAS alarms. When NR + LTE bands are all active with MIMO, the remote unit DSP can bottleneck before the air interface saturates. Fix: review remote unit capacity spec vs actual band config. Add more remote units to split load, or reduce MIMO layers on lower-priority bands.

ONE Platform NR Incompatibility: NR C-Band fails to pass through ONE platform. Signal present at headend input but absent at remote output. Fix: verify ONE model against Corning NR compatibility matrix. May require hardware swap to Everon platform.

Fiber Distance Limitation: Remote units at end of long runs show degraded performance. Everon max ~20km, ONE max ~15km. Actual limit depends on fiber quality and connector count. Fix: verify fiber loss budget, add optical amplifier if needed, reduce connector count (each adds ~0.3-0.5 dB).

## 7.6 SOLiD (ALLIANCE, GENESIS)

SOLiD is strong in transit, transportation, and outdoor venues. GENESIS supports neutral host (multi-operator) deployments. ALLIANCE remote units are rated for outdoor installation.

7.6.1 Common Failure Patterns

Outdoor ARU Moisture Ingress: ALLIANCE Remote Unit shows intermittent faults, worse during rain. VSWR fluctuates on Ericsson side. Even IP67-rated units can fail if gaskets aren't properly seated after maintenance. Fix: inspect enclosure seals/gaskets, re-seat all connectors, apply dielectric grease to RF connections, verify drain holes clear.

Neutral Host Isolation Issues (GENESIS): One carrier's performance degrades when another carrier's traffic increases. Digital processing should isolate carriers but can bleed under high load on older firmware. Fix: update GENESIS firmware, verify per-carrier isolation settings.

SFP Compatibility: Link between headend and ARU fails after SFP replacement. SOLiD requires specific SFP part numbers - third-party SFPs may not be recognized. Fix: use only SOLiD-approved SFP modules from their qualified vendor list.

Headend Fan Failure: Thermal alarm, throughput gradually degrades as unit throttles. Fan module failure triggers thermal protection, reducing output power. Fix: replace fan module (hot-swappable on ALLIANCE), verify cabinet HVAC, check intake filters.

## 7.7 Universal DAS Throughput Symptom Matrix

Use this matrix to quickly identify the most likely cause of throughput issues across any DAS vendor:

Low DL Throughput - All Bands

Probable causes: DAS headend input signal too low (check RRU output), remote unit output power too low (check gain settings), excessive path loss from remote to UE, remote unit DSP overload (too many bands/MIMO layers).

Low DL Throughput - Single Band Only

Probable causes: per-band gain setting incorrect on headend, band-specific filter issue, Ericsson cell for that band in degraded mode, bandwidth allocation reduced for that band.

Low UL Throughput with Normal DL

Probable causes: DAS uplink gain set too low (asymmetric gain is common), PIM causing elevated RSSI at RRU (check AMOS RSSI values > -106 dBm for PCS/AWS or > -109 dBm for N2500), DAS noise figure too high, UE at cell edge with insufficient uplink link budget.

Throughput Drops Under Load

Probable causes: DAS remote unit DSP/capacity limitation, headend composite power limit reached, Ericsson scheduler limiting per-UE allocation (check PRB utilization), DAS backhaul bottleneck between headend and remotes.

## 7.8 DAS Escalation Path

Level 1: Check Ericsson RAN side (CPRI, cells, alarms) - IX team

Level 2: Check DAS headend status and alarms - DAS vendor technician

Level 3: Check DAS distribution path to remote units - DAS vendor technician

Level 4: Joint Ericsson + DAS vendor troubleshooting at integration point

Level 5: Vendor engineering escalation (CommScope / JMA / ADRF / Corning / SOLiD)

ESCALATION TIP: When throughput issues cannot be resolved at L1-L2, request a joint troubleshooting session with both the Ericsson IX team and the DAS vendor technician present. The integration point between RRU output and DAS headend input is where most finger-pointing occurs. Having both parties measure simultaneously at the handoff point eliminates ambiguity.

# 8. Comprehensive Glossary

| Term | Definition & Relevance |
| --- | --- |
| 3GPP | 3rd Generation Partnership Project: standards body defining LTE/NR specifications |
| ADC | Analog-to-Digital Converter: converts RF signal to digital at UE receiver. Saturation causes clipping and SINR degradation |
| AGC | Automatic Gain Control: adjusts receiver gain to keep signal in ADC dynamic range |
| BDA | Bi-Directional Amplifier: amplifies DL and UL signals in DAS systems |
| BLER | Block Error Rate: ratio of erroneously received transport blocks to total blocks |
| CA | Carrier Aggregation: combining multiple component carriers for higher bandwidth |
| C-DRX | Connected Mode Discontinuous Reception: UE sleep cycles during active connection |
| CIQ | Configuration Information Questionnaire: engineering document defining all cell parameters |
| CORESET | Control Resource Set: NR control channel allocation in frequency/time domain |
| CQI | Channel Quality Indicator: UE-reported metric mapping to MCS (0-15) |
| CRS | Cell-specific Reference Signal: LTE pilot signals used for channel estimation and RSRP measurement |
| CSI-RS | Channel State Information Reference Signal: NR reference signal for channel estimation |
| DAS | Distributed Antenna System: network of spatially separated antenna nodes providing indoor coverage |
| DCNR_RESTRICTION | Dual Connectivity NR Restriction: network flag enabling/disabling EN-DC for the UE |
| EARFCN | E-UTRA Absolute Radio Frequency Channel Number: LTE frequency identifier |
| eMBB | enhanced Mobile Broadband: 5G NR slice optimized for high throughput |
| EN-DC | E-UTRA NR Dual Connectivity: NSA architecture with LTE master + NR secondary node |
| ENDC Total Tx Pwr | Combined LTE + NR uplink transmit power in NSA mode |
| EVM | Error Vector Magnitude: measure of modulation accuracy. Must be <8% for 256QAM |
| HARQ | Hybrid Automatic Repeat Request: error recovery via retransmission combining |
| LDPC | Low-Density Parity-Check: NR forward error correction coding (replacing LTE Turbo codes) |
| MCS | Modulation and Coding Scheme: determines modulation order and code rate per transport block |
| MIMO | Multiple Input Multiple Output: spatial multiplexing using multiple antennas |
| MOP | Method of Procedure: standardized testing methodology and pass/fail criteria |
| NR-ARFCN | NR Absolute Radio Frequency Channel Number: 5G NR frequency identifier |
| NR_ANT MAX/MIN RSRP | Strongest/weakest NR receive antenna RSRP (diagnoses antenna path health) |
| NR_BW | NR channel bandwidth in MHz |
| NR_BLER | NR Block Error Rate for the NR leg of EN-DC or SA connection |
| NR_CDRX | NR Connected Mode DRX state (Active/Inactive) |
| NR_DL_Scheduling | Percentage of NR DL slots allocated for data transmission |
| NR_Num_CC | Number of NR Component Carriers active (1 = no NR CA) |
| NR_PCI | NR Physical Cell Identity (0-1007) |
| NR_RX0-RX3 | Individual NR receive chain RSRP values (-140 = no signal) |
| NR_SB Status | NR Split Bearer status: LTE+NR for active EN-DC, -- for standalone |
| NR_SCS | NR Subcarrier Spacing: 15/30/60/120 kHz (numerology 0/1/2/3) |
| NR_SLICE/PDU | Active network slice and PDU session identifier |
| NR_Tx Phy Channel Type | PUCCH (control only) or PUSCH (data active) on NR uplink |
| OLLA | Outer Loop Link Adaptation: algorithm adjusting MCS target based on BLER feedback |
| PCI | Physical Cell Identity: unique cell identifier derived from PSS + SSS |
| PDCCH | Physical Downlink Control Channel: carries scheduling grants and resource allocation |
| PDSCH | Physical Downlink Shared Channel: carries DL user data |
| PIM | Passive Intermodulation: spurious signals from non-linear junctions in passive components |
| PLMN | Public Land Mobile Network: MCC+MNC identifying the operator (310-260 = T-Mobile) |
| PRB | Physical Resource Block: 12 subcarriers x 1 slot (minimum scheduling granularity) |
| PUCCH | Physical Uplink Control Channel: carries UCI (ACK/NACK, CQI, SR) |
| PUSCH | Physical Uplink Shared Channel: carries UL user data |
| RSRP | Reference Signal Received Power: average power per RS RE (dBm) |
| RSRQ | Reference Signal Received Quality: N*RSRP/RSSI (dB) |
| RSSI | Received Signal Strength Indicator: total wideband received power |
| SCS | Subcarrier Spacing: frequency spacing between OFDM subcarriers |
| SINR/SNR | Signal-to-Interference-plus-Noise Ratio (dB) |
| SSB | Synchronization Signal Block: NR broadcast signal for initial access and measurements |
| TAC | Tracking Area Code: identifies tracking area for paging and mobility |
| TPC | Transmit Power Control: closed-loop UL power adjustment commands |
| UPPERLAYER_IND_R15 | Upper layer indication for NR capability: Support = ENDC capable |
| VSWR | Voltage Standing Wave Ratio: measure of impedance mismatch causing reflected power |
| BWP | Bandwidth Part: a contiguous subset of PRBs within a carrier bandwidth, configured per UE to allow different devices (eMBB vs RedCap) to use different bandwidths simultaneously on the same carrier. |
| eRedCap | Enhanced Reduced Capability (3GPP Release 18): 5G IoT device profile with max 5 MHz bandwidth and 10 Mbps peak DL/UL — targets ultra-low-cost sensors. Requires 5G SA core. |
| LTM | L1/L2 Triggered Mobility (3GPP Release 18): Lower-layer handover signaling that reduces mobility interruption from 50-90 ms (L3 HO) to near-zero by reusing beam management procedures. Key for maintaining throughput during speed tests near cell boundaries. |
| NR-DC | NR Dual Connectivity (SA architecture): two NR gNBs (MN + SN) connected via 5GC with no LTE anchor. Enables mid-band + mmWave aggregation on pure 5G SA networks. T-Mobile achieved 2.2 Gbps UL world record using NR-DC (Nov 2024). |
| O-RAN | Open Radio Access Network: disaggregated RAN architecture with standardized interfaces. O-RAN Split 7.2x fronthaul imposes 100 µs latency budget — critical constraint for DAS deployments using open fronthaul transport. |
| RedCap | Reduced Capability (3GPP Release 17): 5G NR IoT device profile with max 20 MHz bandwidth (FR1), 1-2 Rx branches, no CA support. Peak 226 Mbps DL / 120 Mbps UL. Requires 5G SA core. Target use cases: wearables, industrial sensors, surveillance cameras. |
| 256QAM UL | 256 Quadrature Amplitude Modulation Uplink: 8 bits/symbol UL modulation (3GPP TS 38.214 PUSCH MCS table). Requires UL SINR >28 dB at gNB receiver. Supported on Snapdragon X80+ UEs. Provides up to 33% UL throughput gain vs 64QAM. Network-activated via RRC signaling. |
| VoNR | Voice over NR: IMS/SIP-based voice carried natively on 5G NR using EVS/AMR-WB codecs. Uses 5QI=1 DRB (RLC-UM). With ROHC header compression: ~17 kbps per call overhead. Concurrent VoNR + data tests may marginally reduce eMBB PRB availability. |

FR2: Frequency Range 2. 3GPP designation for mmWave bands above 24 GHz (n258: 24.25–27.5 GHz, n260: 37–40 GHz, n261: 27.5–28.35 GHz).

L1-RSRP: Layer 1 Reference Signal Received Power. Per-beam RSRP measurement used for mmWave beam management and LTM (L1/L2 Triggered Mobility) triggering.

LTM: L1/L2 Triggered Mobility. 3GPP Release 18 feature enabling 20–30 ms cell switching via MAC CE signaling, faster than traditional RRC (L3) handover.

BWP: Bandwidth Part. A contiguous set of PRBs within the channel bandwidth. UEs switch between BWPs for power saving. Narrow BWP = lower throughput.

DAA: Dynamic Antenna Adaptation. Network-controlled MIMO layer reduction per BWP for UE power saving. Can reduce throughput proportionally to layer reduction.

K0: Slot offset between PDCCH scheduling grant and PDSCH data transmission. K0 > 0 enables cross-slot scheduling for UE power saving with minimal throughput impact.

mmWave: Millimeter Wave. Radio frequencies in the 24–100 GHz range. Very high capacity but limited range and penetration. Used in DAS for high-density venues.

VoNR: Voice over NR. 5G SA voice service using EVS codec over dedicated GBR QoS flow. Eliminates LTE fallback during calls, preserving NR data throughput during voice calls.

SRS: Sounding Reference Signal. UL signal used for channel estimation, MIMO precoding, and (from 3GPP Rel-16) UE positioning. Default allocation: 4 PRBs.

2.21  NR Positioning & SRS for Positioning — Throughput Overhead

2.21.1  Definition & 3GPP Specification

Sounding Reference Signals (SRS) are UL pilot signals transmitted by the UE and received at the gNB for channel estimation, MIMO precoding feedback, and — from 3GPP Release 16 onward — UE positioning. SRS-based positioning leverages the UL SRS to enable base stations to calculate Time Difference of Arrival (TDoA), Angle of Arrival (AoA), and Round-Trip Time (RTT) measurements. The Location Management Function (LMF) in 5GC aggregates these measurements to compute sub-meter positioning accuracy (< 1 m horizontal for 90% of UEs per 3GPP Release 17 targets; < 0.2 m for IIoT use cases).

SRS is configured per UE via RRC signaling (srs-Config IE in 3GPP TS 38.331). Key parameters include: SRS periodicity (1–2560 ms), SRS bandwidth configuration (C_SRS 0–63 mapping per 3GPP TS 38.211 Table 6.4.1.4.3-1), number of SRS ports (1, 2, or 4), and SRS resource type (periodic, semi-persistent, aperiodic). Aperiodic SRS (triggered by DCI) causes an instantaneous UL overhead spike that can affect PUSCH scheduling in the same TTI.

Samsung service mode: SRS activity is not directly visible as a named field. Indirect indicators: NR_Tx Phy Channel Type switching PUSCH↔PUCCH, elevated NR_UE TX Power during SRS bursts, and reduced NR_DL_Scheduling percentage during high-frequency aperiodic SRS occasions.

2.21.2  UL Throughput Impact of SRS

SRS transmissions occupy UL resources (PRBs and OFDM symbols) that cannot simultaneously carry PUSCH data. The overhead depends on SRS configuration:

| SRS Type | Typical Period | UL Overhead | DAS Impact |
| --- | --- | --- | --- |
| Periodic SRS | 5–160 ms | < 1% per port | Negligible |
| Semi-Persistent SRS | 40–640 ms | < 0.5% | Negligible |
| Aperiodic SRS (1 burst) | On-demand (DCI) | 2–5% in burst TTI | Momentary UL dip |
| Positioning SRS (Rel-16+) | 1–160 ms (config) | 1–4 symbols/slot | Up to 5% at 1 ms period |

KEY INSIGHT: In DAS environments, the gNB may trigger aperiodic SRS more frequently to update MIMO precoding weights due to multipath variation between remote units. This can increase SRS overhead by 2–3× compared to macro deployments. If UL throughput is systematically 5–10% below theoretical maximum without obvious BLER or TX power issues, excessive SRS periodicity is a candidate cause.

2.21.3  DAS-Specific Positioning Considerations

DAS deployments offer a unique advantage for UL positioning: multiple remote units (RUs) distributed throughout a venue act as multiple Transmission/Reception Points (TRPs). The gNB or LMF can collect SRS measurements from each RU independently, enabling multi-TRP RTT or TDOA positioning with higher accuracy than single-antenna macro deployments. 3GPP Release 16/17 defined multi-TRP SRS positioning as a key use case for indoor industrial IoT.

However, for DAS speed test validation, positioning measurements running concurrently with speed tests can cause resource contention. If the network is configured with high-frequency periodic SRS for location services (common in enterprise private network deployments), this competes with PUSCH data scheduling.

2.21.4  Troubleshooting: SRS-Induced UL Throughput Loss

Symptoms: UL throughput 5–15% below formula-predicted maximum; UL BLER normal (< 2%); TX Power nominal.

Step 1: Check NR_Tx Phy Channel Type. If it alternates between PUSCH and PUCCH more than expected during continuous UL traffic, SRS transmissions may be interrupting PUSCH scheduling windows.

Step 2: Request network configuration from the operator or CIQ. Confirm srs-Config periodicity. If srs-periodicity < 5 ms for positioning SRS, escalate to RAN engineer to increase to 10–20 ms for speed test windows.

Step 3: Compare UL throughput with NR Tx Power nominal and BLER < 2%. If UL gap persists, cross-reference SRS overhead calculation: SRS overhead (%) = (SRS symbols per slot × SRS BW PRBs) / (Total UL symbols × Total PRBs) × 100

FIX: For validation testing, request temporary SRS periodicity increase (e.g., 40 ms → 160 ms) from the RAN team. Document the change and revert after testing. If multi-TRP positioning is active, confirm that positioning SRS is on a separate resource pool from data SRS.

2.22  Multi-Operator DAS (Neutral Host) — MOCN/MORAN & Throughput Implications

2.22.1  Definition & Architecture Types

A Neutral Host DAS is a shared infrastructure model where a single physical DAS system serves multiple Mobile Network Operators (MNOs). Two primary 3GPP-defined architectures exist for spectrum and RAN sharing:

| Architecture | Spectrum | Core Network | RAN Hardware |
| --- | --- | --- | --- |
| MORAN (Multi-Operator RAN) | Separate per operator | Separate per operator | Shared RAN, separate radios/carriers |
| MOCN (Multi-Operator Core Network) | Shared (e.g., CBRS 3.5 GHz) | Separate per operator | Fully shared RAN + single radio chain |
| Traditional Licensed DAS (T-Mobile) | Licensed T-Mobile spectrum only | T-Mobile 5GC/EPC | Single-operator RUs, T-Mobile PLMN only |

MOCN in shared spectrum (CBRS 3.5 GHz / Band n48) broadcasts multiple PLMN IDs over the same radio hardware. Each operator's subscribers attach to their own PLMN, and traffic is routed to the operator's separate core. MORAN uses separate carrier frequencies per operator but shares passive DAS infrastructure (cables, splitters, antennas). From the RF perspective, MORAN behavior is identical to a single-operator DAS for each individual carrier.

2.22.2  T-Mobile Position on Neutral Host (2025)

T-Mobile has withdrawn from MOCN-based CBRS neutral host deployments, citing superior performance of its licensed mid-band spectrum (n41 2.5 GHz, n77 3.7 GHz). T-Mobile's neutral host strategy relies on licensed spectrum, with CBRS playing only a selective complementary role. As a result, T-Mobile DAS sites validated under the T-Mobile DAS program use licensed spectrum only (B2, B4, B5, B12, B25, B41, B66, B71 for LTE; n25, n41, n71, n77/n78 for NR) and will present as a single PLMN (310-260 or 311-490 for T-Mobile or Metro/MVNOs).

WARNING: If a Samsung service mode screenshot shows a PLMN other than T-Mobile (310-260 / 311-490), the UE has camped on a co-located carrier's infrastructure. Confirm correct PLMN before capturing any speed test result. A wrong PLMN invalidates the measurement for T-Mobile DAS validation.

2.22.3  Throughput Sharing in MORAN DAS

In MORAN deployments, multiple operator carriers share passive DAS infrastructure. Each operator uses its own spectrum, so there is no resource-level sharing of PRBs between operators. However, passive component intermodulation (PIM) between multiple carriers co-located on the same DAS is a critical concern:

PIM Risk: Two strong DL carriers (e.g., T-Mobile n41 + AT&T n77) on the same DAS can generate 3rd-order intermodulation products that fall in the UL receive band of one or both operators. This manifests as elevated SINR degradation and increased BLER on the affected UL carrier.

Threshold: PIM products must be < -107 dBm (3GPP TS 38.104) to avoid PUSCH interference. VSWR > 1.5:1 on any DAS passive component significantly increases PIM risk.

DL Power Splitting: In a MORAN DAS with N carriers on the same antenna, each carrier's DL signal is split by the passive combiner. A 2-carrier combiner introduces ~3.5 dB loss per port; 4-carrier: ~6 dB. This must be accounted for in the DAS link budget (RSRP will be 3.5–6 dB lower than single-operator design).

2.22.4  Troubleshooting: Neutral Host DAS Issues

Step 1: Confirm T-Mobile PLMN (310-260) in Samsung service mode before proceeding with any measurement.

Step 2: If SINR is lower than expected despite good RSRP, check for PIM. Measure UL noise floor via UE TX Power behavior: if UE is transmitting at maximum power (23 dBm) despite good RSRP, UL interference (possibly PIM from co-located carrier) is suppressing SINR.

Step 3: For MORAN sites, verify the T-Mobile link budget accounts for combiner insertion loss. A 2-carrier MORAN DAS should show RSRP ~3 dB lower than a single-operator design at equivalent distance.

FIX: PIM: Replace passive components failing PIM testing (> -107 dBm threshold). Link budget: Adjust T-Mobile head-end power to compensate for combiner insertion loss. PLMN confusion: Confirm UE is not using automatic network selection that may prefer a stronger non-T-Mobile PLMN.

KEY INSIGHT: MORAN DAS sites are increasingly common in large venues (stadiums, airports, convention centers) where multiple carriers share infrastructure cost. The T-Mobile validation team must confirm single-operator RSRP budget before attributing low RSRP to coverage gaps — the MORAN combiner loss is expected and should be reflected in the CIQ link budget, not treated as a failure.

2.23  VoNR — Impact on Concurrent Data Throughput During Voice Calls

2.23.1  VoNR Architecture & QoS Configuration

Voice over New Radio (VoNR) is the 5G SA native voice service, defined in 3GPP TS 23.501 and TS 26.114. VoNR requires a 5G Standalone (SA) core (5GC) and uses IMS (IP Multimedia Subsystem) for call setup via SIP. Unlike CSFB or VoLTE-over-LTE fallback, VoNR eliminates the need for LTE as a voice anchor, enabling pure NR calls.

VoNR uses two QoS flows on separate Data Radio Bearers (DRBs):

| QoS Flow | 5QI | Bearer Type | RLC Mode | Purpose |
| --- | --- | --- | --- | --- |
| Voice (RTP/EVS) | 1 | GBR (Guaranteed Bit Rate) | RLC-UM | EVS voice frames, 6–128 kbps |
| SIP Signaling | 5 | Non-GBR | RLC-AM | Call setup/teardown, DTMF |
| Data (eMBB) | 9 | Non-GBR | RLC-AM | Internet data (speed test) |

The GBR DRB for 5QI=1 (voice) reserves a minimum guaranteed bitrate at the gNB scheduler. With EVS codec at 12.65 kbps (wideband) and ROHC header compression, the actual air interface resource consumption is approximately 1 PRB per 5 ms TDD slot for the voice DRB. This is a small but non-zero deduction from the scheduling pool available for data.

2.23.2  Quantified Throughput Impact of Active VoNR Call

VoNR voice DRB scheduling overhead on concurrent data (eMBB DRB, 5QI=9):

| EVS Codec Mode | Voice Bitrate | PRB Overhead (100 MHz NR) | DL Throughput Impact |
| --- | --- | --- | --- |
| EVS-NB (narrowband) | 2.4–12.65 kbps | < 0.1% PRB overhead | < 1% DL loss |
| EVS-WB (wideband) | 12.65 kbps (primary) | ~0.1–0.2% overhead | < 1% DL loss |
| EVS-SWB (super-wideband) | 24.4–48 kbps | ~0.3% overhead | ~1–2% DL loss |
| AMR-WB (fallback) | 12.65 kbps | ~0.2% overhead | < 1% DL loss |

KEY INSIGHT: Direct PRB overhead from VoNR voice traffic is negligible (< 2%) and should NOT cause measurable throughput degradation during DAS speed test validation. If significant throughput drops (> 5%) are observed during a VoNR call, the root cause is NOT the voice codec overhead — look for scheduler configuration issues, QoS preemption settings, or incorrect GBR reservation parameters in the CIQ.

2.23.3  DAS Implications: VoNR vs VoLTE Architecture Impact

On NSA (EN-DC) deployments, voice is handled via VoLTE on the LTE anchor (4G DRB). The NR secondary cell carries only data (eMBB 5QI=9). This means active VoLTE calls on EN-DC have zero impact on NR throughput, but may reduce LTE anchor scheduling for SRB/UCI, mildly affecting EN-DC stability.

On NR SA deployments with VoNR, both voice and data share the NR carrier. The GBR scheduling for VoNR takes priority over non-GBR data in congested conditions. However, with 100 MHz NR bandwidth providing ~1.5 Gbps peak DL scheduling capacity and EVS consuming < 200 kbps, the capacity impact is effectively zero under normal DAS validation conditions.

2.23.4  Samsung Service Mode: VoNR Indicators

NR_SB Status: Shows 'LTE+NR' for EN-DC (VoLTE on LTE), '--' for NR SA. NR SA with VoNR shows '--' for NR_SB Status but active NR_RRC State = RRC_CONNECTED.

NR_Tx Phy Channel Type: Shows PUSCH during active VoNR call (UL voice RTP packets on NR PUSCH). If PUCCH-only during a VoNR call, UL voice packets may not be transmitting correctly.

ENDC Total Tx Pwr: On EN-DC, voice (LTE) and data (NR) TX powers are separate. High LTE TX power during VoNR may indicate poor LTE anchor coverage limiting EN-DC stability.

NR_SLICE/PDU: VoNR and data use different 5QI flows but may share the same PDU session. Confirm eMBB slice (SST=1) is active for data DRB.

2.23.5  Troubleshooting: Throughput Drop During VoNR Calls

Step 1: Confirm architecture. NR SA + VoNR call active? Or EN-DC + VoLTE? If EN-DC: NR throughput should be unaffected by voice. If NR SA: minimal impact expected (< 2%).

Step 2: If throughput drops > 5% during VoNR, check for GBR over-provisioning. Some networks configure GBR for 5QI=1 at 256 kbps or higher (legacy settings). This pre-empts non-GBR data scheduling more aggressively than needed.

Step 3: Verify ROHC is configured (RRC: pdcp-Config headerCompression). Without ROHC, RTP+UDP+IP headers add ~40 bytes per 20 ms voice packet (200 bytes/s overhead × many UEs = measurable UL PUSCH resource consumption on dense DAS sites).

FIX: For speed test validation: avoid simultaneous VoNR calls during throughput measurement. If unavoidable, document that a VoNR call was active and note expected < 2% impact. For production issues: verify EVS codec is negotiated (not AMR-NB fallback) and ROHC is enabled. Escalate to RAN if 5QI=1 GBR reservation exceeds 128 kbps per UE on a 100 MHz carrier.

SRS (Positioning)
Sounding Reference Signal configured for UE positioning (3GPP TS 38.214 / 38.211). UE transmits SRS in UL; gNBs/TRPs measure timing and angle for location computation. Overhead: < 1% periodic, up to 5% with high-frequency aperiodic SRS triggers. Relevant to DAS multi-TRP positioning and UL throughput analysis.

MOCN
Multi-Operator Core Network: RAN sharing architecture where multiple MNOs share the same radio hardware and spectrum but maintain separate core networks. Common in CBRS neutral host DAS. Each operator broadcast as a distinct PLMN. T-Mobile uses licensed-spectrum neutral host, not CBRS MOCN, as of 2025.

MORAN
Multi-Operator RAN: RAN sharing where multiple MNOs share passive DAS infrastructure (cables, antennas, splitters) but each operator uses its own licensed carrier frequency. Each operator's throughput is independent but PIM between co-located carriers is a risk.

Neutral Host DAS
A shared DAS infrastructure model serving multiple MNOs from a single physical installation. Reduces venue deployment cost. T-Mobile supports licensed-spectrum neutral host (MORAN) but not CBRS MOCN as of 2025.

EVS
Enhanced Voice Services: 3GPP-specified adaptive bitrate codec for VoLTE/VoNR. Supports 2.4–128 kbps, narrowband through super-wideband audio. Primary VoNR codec on T-Mobile 5G SA network. At 12.65 kbps WB with ROHC, PRB overhead < 0.2% on 100 MHz NR carrier.

ROHC
RObust Header Compression (IETF RFC 5795): PDCP-layer IP/UDP/RTP header compression for VoLTE/VoNR. Reduces RTP header from ~40 bytes to 1–3 bytes per packet, critical for efficient VoNR scheduling. Configured in RRC pdcp-Config headerCompression IE.

GBR DRB
Guaranteed Bit Rate Data Radio Bearer: A DRB with a minimum guaranteed scheduling rate enforced by the gNB scheduler. Used for VoNR (5QI=1) voice traffic. Pre-empts non-GBR (eMBB data) DRBs in congestion. GBR for EVS voice typically 12.65–128 kbps; negligible impact on 100 MHz NR throughput.

5QI
5G QoS Identifier: A scalar value (1–255) defining the QoS characteristics of a QoS flow. 5QI=1: GBR conversational voice (VoNR). 5QI=5: IMS signaling. 5QI=9: default non-GBR eMBB data (speed test traffic). Defined in 3GPP TS 23.501 Table 5.7.4-1.

TRP
Transmission/Reception Point: A logical or physical unit in the RAN that transmits and receives NR signals. In DAS, each remote unit (RU) can function as a separate TRP for multi-TRP MIMO or SRS-based positioning. Multi-TRP is defined in 3GPP TS 38.214 for Rel-16+ DAS scenarios.

LMF
Location Management Function: 5GC network function responsible for UE positioning. Receives SRS measurements from gNB/TRPs, applies TDOA/AoA/RTT algorithms, and reports UE location. Defined in 3GPP TS 23.501. Relevant to DAS sites with positioning SRS configured.

# 9. MS2 KPI & Counter Correlation Framework

This section establishes the causal chain between RF parameters observed in Samsung service mode screenshots, CIQ-configured cell parameters, and the Ericsson MS2 (Managed Services 2) KPI and counter framework. Understanding this mapping is essential for an Expert RF Engineer because it bridges the gap between field-level RF measurements and network-level performance reporting. A degraded RF parameter at the UE does not exist in isolation—it propagates through the RAN protocol stack and manifests as degraded KPI counters visible in ENM/ENIQ reporting.

The MS2 KPI framework is organized into six functional domains: Availability (AVL), Accessibility (ACC), Retainability (RET), Capacity (CAP), Mobility (MOB), and Power/Interference (PWR). Each domain contains KPIs derived from Ericsson PM counters collected at the NRCELLDU, NRCELLCU, and EUTRANCELLFDD managed objects.

## 9.1 NR KPI Correlation to RF Parameters

### 9.1.1 Throughput KPIs

The following NR throughput KPIs from the MS2 framework are directly impacted by the RF parameters documented in Section 2 of this RAG.

| KPI ID | KPI Name | RF Parameters That Drive This KPI | CIQ Parameters That Impact |
| --- | --- | --- | --- |
| NTM_NR_CAP_03_04_061 | DL Average UE Throughput (Mbps) | RSRP, SINR, BLER, NR_DL_Scheduling, MIMO Rank, Channel BW | dlChannelBandwidth, noOfTxAntennas, TDD slot pattern, MCS table config |
| NTM_NR_CAP_03_04_062 | UL Average UE Throughput (Mbps) | UE TX Power, UL SINR at gNB, BLER, NR_Tx Phy Channel Type | ulChannelBandwidth, TDD UL slot ratio, P_0, alpha (path loss comp) |
| NTM_NR_CAP_03_04_001 | DL MAC Data Volume (MB) | Aggregate of DL throughput x session time. Reflects overall RF health. | Cell BW, MIMO config, CA enablement, scheduling policy |
| NTM_NR_CAP_03_04_002 | UL MAC Data Volume (MB) | Aggregate UL volume. Low value = UL path issues (TX power, TDD ratio, noise figure). | UL BW, TDD pattern, UL power control params |

KPI CORRELATION: When DL Average UE Throughput (NTM_NR_CAP_03_04_061) is below MOP baseline: (1) Verify SINR > 25 dB → if not, check PUSCH Receive Interference Power (NTM_NR_PWR_03_15_010); (2) Verify BLER < 10% → if not, EVM/linearity issue; (3) Verify NR_DL_Scheduling > 60% → if not, EN-DC coordination issue; (4) Cross-check DL MAC Data Volume trending → sudden drops indicate cell-level events.

### 9.1.2 Accessibility KPIs

Accessibility KPIs measure whether UEs can successfully establish connections. In DAS environments, poor RSRP or high interference directly degrades these counters.

| KPI ID | KPI Name | RF Root Cause Chain | DAS-Specific Triggers |
| --- | --- | --- | --- |
| NTM_NR_ACC_03_02_023 | RACH CBRA Success Rate | RSRP < -110 dBm → PRACH not detected. SINR < 3 dB → Msg3 failure. PIM → Msg1 OOC. | DAS UL gain too low, dead antenna path, excessive path loss from remote unit |
| NTM_NR_ACC_03_02_096 | NSA SN Addition Failure Rate | NR RSRP < -100 dBm → SN threshold not met. SINR < 10 dB → SgNB rejects. | NR radio unit off, NR band blocked by DAS, DCNR_RESTRICTION=TRUE |
| NTM_NR_ACC_03_02_010 | NSA SN Addition Total Failures | Spike correlates with NR cell outage or degraded NR RSRP. | X2 interface latency > 10ms, NR license missing, BBU capacity exceeded |
| NTM_NR_ACC_03_02_093 | SA NGBR Data DRB Access Failure Rate | RSRP/SINR below camping threshold → RRC failure. Backhaul → DRB failure. | Core network connectivity, S1/NG interface, gNB overload |
| PMRADIORACBFAILMSG1OOC | RACH Msg1 Out-of-Coverage Failures | UE PRACH preamble too weak. Path loss exceeds UL link budget. | DAS UL NF > 6 dB, remote unit UL amp failure, antenna VSWR exceeding market threshold (≤ 2.1 for 4402/2203, ≤ 1.4 for 8863) |

KPI CORRELATION: RACH CBRA Success Rate below 95% on a DAS cell is a red flag. In a controlled DAS environment, RACH success should exceed 99%. PMRADIORACBFAILMSG1OOC directly maps to UL coverage gaps. If this counter is non-zero, the DAS UL path loss budget is violated.

### 9.1.3 Retainability KPIs

Retainability KPIs measure whether established connections survive. Drops during active sessions correlate with RLF events and RF degradation.

| KPI ID | KPI Name | RF Root Cause Chain | Speed Test Impact |
| --- | --- | --- | --- |
| NTM_NR_RET_03_03_001 | NSA SN Total Drop Rate | NR RSRP < -100 dBm → T310 expires → RLF → SN release. | Any SN drop during speed test invalidates result. |
| NTM_NR_RET_03_03_002 | NSA SN Total Drops (count) | Correlates 1:1 with NR_RLF Count in service mode. | Each drop reduces measured throughput 20-50%. |
| NTM_NR_RET_03_03_029 | SA NGBR Data DRB Total Drop Rate | SA mode: no LTE fallback. RSRP/SINR degradation → RLF → complete interruption. | SA drops more impactful than NSA. |
| NTM_NR_RET_03_03_019 | SA DRB Active Drop Rate | Drops during active data transfer. Maps to mid-session throughput cliffs. | Most critical retainability KPI for DAS validation. |
| NTM_NR_RET_03_03_020 | SA DRB Active Drop Volume | High count on specific cell → sector-specific RF issue. | Track per-sector to isolate hardware issues. |

KPI CORRELATION: NSA SN Total Drop Rate (NTM_NR_RET_03_03_001) exceeding 2% on a DAS cell: check (1) LTE anchor RSRP stability — anchor below -85 dBm triggers SN release; (2) NR SINR trends — below 3 dB for > 200 ms triggers T310; (3) X2/Xn interface health — transport issues cause abnormal SN releases.

### 9.1.4 Interference & Power KPIs

PUSCH Receive Interference Power is the most important network-side indicator for DAS PIM and noise floor issues.

| KPI ID | KPI Name | What It Measures | RF Parameter Correlation |
| --- | --- | --- | --- |
| NTM_NR_PWR_03_15_010 | PUSCH Receive Interference Power (dBm) | UL interference power distribution at gNB receiver. | SINR at UE (inverse): high interference → low UE SINR. PIM and macro ingress raise this. |
| AvgPUSCHSINR [OBSOLETE] | Average PUSCH SINR | gNB-measured UL SINR on PUSCH. | Maps to UE TX Power: low gNB PUSCH SINR → UE TX Power goes positive. |
| AvgPUCCHSINR [OBSOLETE] | Average PUCCH SINR | gNB-measured UL SINR on PUCCH control channel. | Poor PUCCH SINR → scheduling grant failures → reduced DL scheduling. |

KPI CORRELATION: EXPERT RF INSIGHT: PUSCH Receive Interference Power above -105 dBm on a DAS cell is abnormal. In a well-isolated DAS, this should be -110 to -120 dBm. Values above -105 dBm = PIM from DAS passive components, macro ingress, or DAS amplifier oscillation. Cross-validate: NTM_NR_PWR_03_15_010 elevated + UE SINR < 25 dB + UE TX Power positive = confirmed UL interference requiring PIM sweep.

### 9.1.5 Mobility KPIs

In DAS environments with multiple sectors, intra-gNB handover should be near-seamless. Failures indicate PCI planning issues or coverage gaps.

| KPI ID | KPI Name | RF Root Cause When Degraded | CIQ Parameters That Impact |
| --- | --- | --- | --- |
| NTM_NR_MOB_03_05_103 | SA HO Intra-Freq Success Rate | Poor RSRP at cell edge. PCI mod-3 collision. Coverage hole between sectors. | a3offset, hysteresisA3, timeToTriggerA3, PCI allocation |
| NTM_NR_MOB_03_05_113 | SA HO Inter-Freq Success Rate | Target cell RSRP insufficient. Measurement gap config incorrect. | measGapConfig, interFreqCarrierFreqList |
| NTM_NR_MOB_03_05_019F | SA HO Failure Rate | Combined prep + exec failures. High rate → SINR degradation, PCI confusion. | Neighbor relations, PCI planning |
| nr_endc_pscell_succ_rat | ENDC Inter-SgNB PSCell Change | NR RSRP too weak at target during active ENDC. | B1 threshold for NR measurement, PSCell offset |
| nr_endc_intra_sgnb_psc_chg_sr | ENDC Intra-SgNB PSCell Change | Intra-gNB NR cell change. Target NR cell degraded. | Same-gNB NR cell RSRP balance |

KPI CORRELATION: SA HO Intra-Frequency Success Rate should exceed 98% in DAS. Below 95%: walk-test sector boundaries, verify RSRP from both cells > -85 dBm at handover zone. PCI mod-3 collision causes 3 dB SINR degradation at cell edge. Compare Prep Failures (NTM_NR_MOB_03_05_107) vs Exec Failures (NTM_NR_MOB_03_05_111) to isolate measurement vs resource issues.

### 9.1.6 Capacity & User KPIs

| KPI ID | KPI Name | DAS Validation Relevance | RF Impact Chain |
| --- | --- | --- | --- |
| NTM_NR_CAP_03_04_112 | Average Connected User Number | High user count during test → PRB contention → lower per-UE throughput. | More users = lower NR_DL_Scheduling per UE |
| NTM_NR_CAP_03_04_048 | DL Average Active User Number | > 5 active DL users on 100 MHz NR → per-UE throughput reduction. | Scheduler allocates PRBs proportionally |
| NTM_NR_CAP_03_04_050 | UL Average Active User Number | Multiple UL users share TDD UL slots. | TDD UL constraint compounded by multiple UL users |
| NTM_NR_AVL_03_08_001 | NR Cell Availability | Must be 100% during validation. Any downtime = zero throughput. | Radio unit failure, BBU fault, CPRI loss |
| NRDLTrafficVolMACMB | NR DL MAC Traffic Volume (MB) | Post-optimization, DL volume should increase. | Higher SINR → higher MCS → more bits per PRB |

## 9.2 LTE KPI Correlation to RF Parameters

LTE KPIs are critical because: (a) LTE is the anchor for EN-DC—anchor degradation collapses NR throughput, and (b) VoLTE quality determines voice reliability in the venue.

### 9.2.1 LTE Throughput & Traffic KPIs

| KPI ID | KPI Name | RF Parameter Drivers | CIQ Impact |
| --- | --- | --- | --- |
| AverageUEDLThroughputkbps | Avg UE DL Throughput (kbps) | RSRP, SNR, RSRQ, BLER, MIMO rank, CA activation | dlChannelBandwidth, noOfTxAntennas, CA pairs, 256QAM enablement |
| AverageUEULThroughputkbps | Avg UE UL Throughput (kbps) | UE TX Power, UL path loss, UL SINR at eNB | ulChannelBandwidth, P_0, alpha, UL CA pairs |
| DLTrafficVolumeMB | DL Traffic Volume (MB) | Aggregate DL volume = f(throughput, sessions). Cell-level RF health indicator. | Cell BW, CA config, MIMO, scheduling policy |
| ULTrafficVolumeMB | UL Traffic Volume (MB) | Depressed by high TX power, UL CA disabled. | UL BW, UL CA enablement, power control params |

### 9.2.2 VoLTE KPIs (LTE Anchor Voice Quality)

VoLTE drops caused by poor LTE RF also destabilize EN-DC, causing NR SN releases.

| KPI ID | KPI Name | RF Root Cause When Degraded | DAS Action |
| --- | --- | --- | --- |
| VoLTEDropRate | VoLTE Drop Rate (%) | LTE RSRP < -95 dBm → QCI 1 RLF. SINR < 5 dB → PDCCH failure. | Increase DAS LTE output power. Fix PIM. Optimize PCI. |
| VOLTE_Drops_due_to_UELost | VoLTE Drops: UE Lost | Coverage hole → UE loses sync. Most severe RF-caused drop. | Walk-test boundaries. Verify DAS coverage overlap. |
| VOLTE_Drops_due_to_HOExecFail | VoLTE Drops: HO Exec Failure | Target cell RSRP insufficient. PCI confusion. | Verify neighbor relations. Walk-test HO boundary. |
| VOLTE_Drops_due_to_HOPrepFail | VoLTE Drops: HO Prep Failure | Target cell not responding. X2 issue. | Verify X2 link. Check target cell availability. |
| VOLTE_Drops_due_to_TNFailure | VoLTE Drops: TN Failure | Backhaul failure. Not an RF issue. | Check RP/backhaul link, fiber/Ethernet errors. |
| VolteAccessFailureRateGCR | VoLTE Access Failure Rate (GCR) | RSRP too weak for RACH. Congestion. | Ensure LTE RSRP > -85 dBm. Verify GBR availability. |

KPI CORRELATION: VoLTE Drops due to UE Lost should be near zero in DAS. Non-zero values pinpoint coverage holes. Cross-reference with ERABDropduetoUELost—if both show UE Lost drops on the same cell, there is a definitive coverage gap requiring RF fix: increase DAS power, add antenna coverage, or fix dead antenna path.

### 9.2.3 E-RAB (Bearer) KPIs

| KPI ID | KPI Name | RF Correlation | EN-DC Impact |
| --- | --- | --- | --- |
| ERABDropRate | E-RAB Drop Rate (Market) | RF-caused: UE Lost, HO failures. Non-RF: MME, TN failure. | High E-RAB drops on anchor → NR SN releases. |
| ERABInitialAccessFailureRate | E-RAB Initial Access Failure Rate | RSRP < -100 dBm → RACH failure. SNR < 5 dB → Msg4 failure. | Access failure on anchor = no EN-DC possible. |
| ERABDropduetoUELost | E-RAB Drop: UE Lost | Coverage hole or sudden interference spike. | UE Lost on anchor = both LTE + NR terminated. |
| ERABDropduetoHOExecutionFailur | E-RAB Drop: HO Exec Failure | Target cell RSRP/SINR insufficient. | Brief outage. On anchor, NR may or may not survive. |
| ERABDropduetoCellDowntime | E-RAB Drop: Cell Downtime | Not RF—cell administratively/automatically down. | 100% throughput loss. Check radio unit status. |

### 9.2.4 RACH & Access KPIs

| KPI ID | KPI Name | RF Correlation | DAS Diagnostic Value |
| --- | --- | --- | --- |
| RACHCBRASuccessRate | RACH CBRA Success Rate (%) | RSRP determines PRACH detection. < -110 dBm → failure. | Should be > 99% in DAS. Below 95% = UL coverage issue. |
| RACHCBRAFailRate | RACH CBRA Fail Rate (%) | High fail rate + good DL RSRP = UL-specific issue. | Compare with NR RACH for cross-technology UL health. |
| PeakNrOfRRCConnectedUsers | Peak RRC Connected Users | Loading indicator. Peak users during test = scheduling contention. | > 20 users on 20 MHz LTE = congested. |

### 9.2.5 MIMO & Carrier Aggregation KPIs

These LTE KPIs validate whether CIQ-configured MIMO and CA settings are being utilized. Low utilization despite correct CIQ = RF-level issues.

| KPI ID | KPI Name | What It Validates | RF Root Cause if Low |
| --- | --- | --- | --- |
| ClosedLoop_2x2MIMORank2Usage | 2x2 MIMO Rank 2 Usage (%) | Should be > 60% in DAS. | Antenna path imbalance, SINR < 15 dB, high antenna correlation. |
| ClosedLoop_4x4MIMOR3R4Usage | 4x4 MIMO R3/R4 Usage (%) | Requires 4-port antenna and quad-feed DAS. | DAS configured as 2-port (only Rank 2 possible). |
| OverallClosedLoopMIMOUsage | Overall CL MIMO Usage (%) | Should be > 70% in well-configured DAS. | MIMO disabled in CIQ or SINR too low (< 10 dB). |
| CA_Activated2CC_RATIO | CA 2CC Activation Ratio (%) | Should be > 80% when configured. | SCell RSRP too weak (< -100 dBm), SCell not in CIQ. |
| CA_Activated3CC_RATIO | CA 3CC Activation Ratio (%) | Requires 3 LTE bands with CA pairs. | Third carrier not in DAS or CA pairs incomplete. |

KPI CORRELATION: MIMO Rank 2 Usage below 40% on a 2x2-configured DAS cell is diagnostic. Check: (1) NR RX0-RX3 RSRP for > 15 dB imbalance; (2) SINR < 15 dB forcing Rank 1; (3) Antenna correlation in echo-rich environments. CA activation ratios should be cross-referenced with CA-Act field in Samsung service mode.

### 9.2.6 LTE Mobility KPIs

| KPI ID | KPI Name | RF Correlation | DAS Impact |
| --- | --- | --- | --- |
| LTEMobilitySuccessRateInterInt | LTE Mobility Success Rate (%) | Low → coverage gaps at sector boundaries, PCI issues. | Sector overlap zone too narrow or RSRP delta too high. |
| LTEMobilityFailuresInterIntra | LTE Mobility Failures (count) | Per-cell trending identifies problematic sectors. | Each HO failure = throughput interruption. On anchor = potential NR drop. |

## 9.3 CIQ Parameter → KPI Impact Chain

This section maps CIQ-configurable parameters to the KPIs they influence.

### 9.3.1 Power & Coverage Parameters

| CIQ Parameter | What It Controls | KPIs Improved | KPIs Degraded if Misconfigured |
| --- | --- | --- | --- |
| DAS Output Power / Attenuator | Per-antenna RSRP | DL/UL Throughput, RACH Success, MIMO Rank, VoLTE Drop Rate | PUSCH Interference rises if DAS over-drives adjacent sectors |
| noOfTxAntennas / noOfRxAntennas | MIMO order (1, 2, 4) | MIMO Rank Usage, DL Throughput | Set to 1 when DAS supports 2 → 50% throughput loss |
| dlChannelBandwidth | Channel bandwidth (MHz) | DL/UL Throughput (linear scaling) | BW set lower than radio supports → capped throughput |
| PCI allocation | Cell identity | Mobility Success, SINR (no mod-3 collision) | PCI collision → SINR degradation up to 10 dB |
| P_0 (target UL power) | UL power control target | UL Throughput, RACH Success | P_0 too high → UL interference. P_0 too low → insufficient UL SINR. |

### 9.3.2 EN-DC & CA Parameters

| CIQ Parameter | What It Controls | KPIs Directly Impacted | Failure Mode |
| --- | --- | --- | --- |
| DCNR_RESTRICTION (T/F) | EN-DC enablement | NSA SN Addition (0 attempts if TRUE) | TRUE on DAS cell = NR never activates |
| UPPERLAYER_IND_R15 | NR capability indicator | NSA SN Addition Failure Rate (100% if Not Supported) | eNB not advertising NR capability |
| LTE CA Pairs (SCell config) | Carrier Aggregation | CA Activation Ratios, DL Throughput | Missing CA pair = CA never activates |
| SCellDeactivationTimer | SCell active duration | CA Activation Ratios, DL stability | Timer < 640 ms → SCell deactivates between bursts |
| X2/Xn Interface Config | Inter-node connectivity | NSA SN Addition, PSCell Change, NR Drop Rate | Latency > 10 ms → SN addition timeout |

### 9.3.3 TDD & Scheduling Parameters

| CIQ Parameter | What It Controls | KPIs Impacted | DAS Note |
| --- | --- | --- | --- |
| TDD Slot Pattern (DDDSU) | DL/UL time allocation | DL/UL Throughput ratio, UL MAC Volume | DDDSU: ~70% DL, ~20% UL. Sets hard UL ceiling. |
| BWP Inactivity Timer | Narrow BWP switch | DL Throughput (first seconds of test) | Short timer = throughput spikes at test start. |
| C-DRX Config | UE sleep cycles | DL/UL Throughput (5-15% reduction) | Document C-DRX state during speed test. |
| SRS Periodicity | UL sounding interval | UL Throughput (overhead increases with shorter period) | SRS < 5 ms → up to 5% UL throughput loss. |

## 9.4 Expert RF Engineer’s KPI Monitoring Playbook for DAS Validation

This playbook provides a structured approach for using MS2 KPIs to validate DAS RF performance before, during, and after speed test campaigns.

### 9.4.1 Pre-Test KPI Baseline (T-24h)

Pull these KPIs for the past 24 hours from ENM/ENIQ to establish a baseline before conducting speed tests.

| KPI to Check | Expected (DAS) | Red Flag | Action |
| --- | --- | --- | --- |
| NR Cell Availability | 100% | < 99.9% | Cell had downtime. Check radio unit alarms. |
| NSA SN Addition Failure Rate | < 2% | > 5% | EN-DC not establishing. Check X2, NR cell. |
| NSA SN Total Drop Rate | < 1% | > 3% | NR link unstable. Check RSRP/SINR. |
| RACH CBRA Success (NR) | > 99% | < 95% | UL coverage issue. Check DAS UL gain. |
| RACH CBRA Success (LTE) | > 99% | < 95% | LTE UL coverage issue. |
| PUSCH Interference Power | < -110 dBm | > -105 dBm | PIM or macro ingress. PIM sweep needed. |
| E-RAB Drop Rate | < 0.5% | > 1% | LTE bearer instability. Fix before NR testing. |
| VoLTE Drop Rate | < 0.5% | > 1% | Voice quality issue on anchor. |

### 9.4.2 During-Test KPI Correlation

While speed tests are running, correlate Samsung service mode observations with near-real-time MS2 KPIs (15-minute ROP granularity).

| Service Mode Observation | MS2 KPI | Correlation Logic |
| --- | --- | --- |
| RSRP < -75 dBm at antenna end | DL Avg UE Throughput depressed | Weak RSRP → lower CQI → lower MCS → lower KPI |
| SINR < 25 dB | PUSCH Interference Power elevated | Low SINR = high interference at gNB side |
| NR_RLF Count > 0 | NSA SN Total Drops increment | Each RLF = one pmEndcRelUeAbnormal increment |
| UE TX Power positive | UL Avg Throughput depressed | Power-limited UE → lower UL MCS at gNB |
| MIMO = 1 (should be 2) | MIMO Rank 2 Usage = 0% | CIQ says 2x2 but Rank 1 observed. Check antenna paths. |
| CA-Act missing SCells | CA Activation Ratio low | SCell not activating despite CIQ config. |
| NR_DL_Scheduling < 60% | Connected User count high | Resource contention. Check NTM_NR_CAP_03_04_112. |

### 9.4.3 Post-Test KPI Validation (T+24h)

After RF optimization, pull the same KPIs for the next 24 hours and compare against pre-test baseline. This provides network-level confirmation that field RF improvements translated into KPI gains.

| KPI | Expected Post-Optimization Trend | If No Improvement |
| --- | --- | --- |
| DL/UL Avg UE Throughput | Increase 10-50% depending on fix | May be masked by user load. Check same time-of-day. |
| NSA SN Drop Rate | Decrease toward < 1% | LTE anchor may still be unstable. |
| RACH Success Rate | Increase toward > 99% | UL gain improvement may need time. |
| PUSCH Interference Power | Decrease 3-10 dB after PIM remediation | PIM source not fully resolved. Second sweep. |
| MIMO Rank 2 Usage | Increase toward > 60% after antenna fix | Path still imbalanced. Verify RX chains. |
| CA Activation Ratios | Increase toward > 80% after SCell fix | SCell RSRP still marginal. |

KPI CORRELATION: PLAYBOOK SUMMARY: The three-phase KPI cycle (Pre-Test Baseline → During-Test Correlation → Post-Test Validation) transforms DAS validation from a point-in-time snapshot into continuous performance assurance. Always document pre/post KPI deltas in the site acceptance report alongside speed test screenshots.